XXX平台结构强度改造可行性评估报告

xxxx年xx月xx日

目录

1	概述	1

1.1	平台概况	1

1.2	改造历史	1

1.3	平台的评估结论	1

2	评估依据	2

2.1	规范依据	2

2.2	评估流程	2

2.3	基础数据	2

2.4	载荷变化	3

2.5	环境条件	3

2.5.1	水深	3

2.5.2	波浪	3

2.5.3	海流	4

2.5.4	风	5

2.5.5	海生物	5

2.5.6	飞溅区和腐蚀余量	5

2.5.7	基础冲刷	6

3	分析模型	7

3.1	程序和坐标系统	7

3.2	结构模拟	8

3.3	桩基础结构模拟	8

4	设计水平分析	9

4.1	环境载荷参数	9

4.2	结构动力响应	9

4.3	基本载荷工况	9

4.4	组合载荷工况	16

4.5	分析结果	30

4.5.1	构件名义应力校核	30

4.5.2	节点冲剪应力校核	32

4.5.3	桩基承载力及桩应力校核	34

- 概述

平台概况

（第一部分：来自建设阶段完工文件平台的描述）

例子：xxx平台位于xxx油气田，水深为xxx m，为xx腿综合处理平台，于2006年安装并投产，平台原设计寿命xx年。（数据来自平台的基本信息）

（第二部分：来自历次改造信息里每一个改造项目的信息和结论）

例子：2016年通过产能释放项目增加井槽和立管，结果显示所有杆件UC值小于1.0，桩基承载力安全系数大于1.5，满足规范要求；极限强度分析结果显示最小RSR为2.1，满足规范要求。2020年通过xxx项目增加xxx，结果显示xxx。

（第三部分：来自检验记录最近一次检验（包括定期检验1-N和特殊事件检测）的信息和结论（可在事件名称后增加“描述”一栏））

例子：2018年五年特检报告显示，水下导管架杆件结构完整、未发现凹陷变形与机械损伤，牺牲阳极均在位，连接牢固；焊缝检测未发现缺陷，飞溅区构件测厚结果最小腐蚀厚度0.1mm，最大腐蚀厚度0.5mm，平台水下结构状况良好。

改造历史

（来自历次改造信息里每一个改造项目列表）

平台历次改造清单如下：

| 序号 | 项目名称 | 年份 |
| --- | --- | --- |
| 1 | xxx平台增加调整井 | 2009年 |
| 2 | xxx 油田产能释放 | 2016年 |
| 3 | xxx油田开发工程详细设计项目旧平台改造结构 | 2011年 |
| 4 | xxx平台增加救生筏和逃生软梯安装甲板 | 2016年 |
| 5 | xxx平台A3井增加放空管线 | 2011年 |

平台的评估结论

（来自本次改造的信息和评估结论）

例子：本次改造新增井槽xxx根，立管和电缆xxx条，上部组块增加重量xxx。对平台改造后的整体结构进行设计水平强度分析，结果显示所有杆件和节点UC值小于1.0，桩基承载力安全系数大于1.5，满足规范要求。综合以上结果，认为本次改造可行。（数据来自平台的基本信息）

- 评估依据

规范依据

（该部分内容内置）

- American Welding Society (AWS), D1.1/D1.1M, Structural Welding Code.

- API RP 2A,	Recommended Practice for Planning, Designing, and Constructing Fixed Offshore Platforms – Working Stress Design.

- American Institute of Steel Construction (AISC), Allowable Stress Design.

- API RP 2SIM

- BS7910, Guide to Methods for Assessing the Acceptability of Flaws in Metallic Structures.

- GB 46766-2025 海洋石油固定平台安全规范

- 固定式导管架结构可靠性分析及应用指南，中国船级社

评估流程

（该部分内容内置）

结构评估流程参考API RP 2SIM。分析方法可采用类比法、设计水平分析法、极限强度分析法、其他替代方法，本报告采用设计水平分析。

基础数据

（来自文件管理相应平台的文件）

1）建设阶段完工文件

（1）xxx

（2）xxx

……

2）历次改造文件

（1）xxx

（2）xxx

……

3）检测记录文件

（1）xxx

（2）xxx

……

……

载荷变化

荷载情况如下表：

（来自载荷信息相应平台的载荷信息表，表格一致）

原有结构及改造的结构在模型中进行模拟，由程序自动计算重量。

环境条件

水深

（来自平台所在油气田信息的水深水位信息）

海图水深为122.20 m（该数据来自平台的水深数据），计算中使用的水位（m）如下所示。

| 元素 |  | 相对海图基准面 |
| --- | --- | --- |
|  |  | m |
| 海图基准面 (CD) |  | 0.00 |
| 最高天文潮 (HAT) |  | 2.55 |
| 最低天文潮 (LAT) |  | 0.00 |
| 平均海平面 (MSL) |  | 1.20 |
| 最高水位 | 1年回归周期 | 2.68 |
|  | 50年回归周期 | 2.91 |
|  | 100年回归周期 | 2.96 |
|  | 1000年回归周期 | 3.16 |
| 最低水位 | 1年回归周期 | -0.14 |
|  | 50年回归周期 | -0.43 |
|  | 100年回归周期 | -0.49 |
|  | 1000年回归周期 | -0.55 |

波浪

（来自平台所在油气田信息的波浪信息）

根据平台详细设计的设计基础，平台波浪的主极值与条件极值的参数如下所示。对干净的圆形构件的Cd和Cm值分别取0.65和1.2，对粗糙的圆形构件的Cd和Cm值分别取1.05和1.2。

| 元素 |  | 回归周期 (年) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 10 | 25 | 50 | 100 |
| 主极值 | 有义波高Hs(m) | 7.3 | 10 | 11.2 | 12 | 12.8 |
|  | 波峰高度Crest(m) | 8 | 11 | 12.4 | 13.2 | 14 |
|  | 最大波高Hmax(m) | 12.5 | 17.2 | 19.3 | 20.7 | 21.9 |
|  | 跨零周期Tz(s) | 9.3 | 10.6 | 11.1 | 11.4 | 11.7 |
|  | 谱峰周期Tp(s) | 12 | 13.5 | 14 | 14.4 | 14.8 |
|  | 平均周期Tm(s) | 11.8 | 13.5 | 14.1 | 14.5 | 14.9 |
| 风主极值条件下极值 | 有义波高Hs(m) | 6.5 | 9.2 | 10.6 | 11.7 | 12.5 |
|  | 最大波高Hmax(m) | 11.2 | 15.8 | 18.2 | 20.2 | 21.5 |
|  | 跨零周期Tz(s) | 8.8 | 10.2 | 10.8 | 11.3 | 11.6 |
|  | 谱峰周期Tp(s) | 11.5 | 13.1 | 13.8 | 14.3 | 14.6 |
|  | 平均周期Tm(s) | 11.2 | 13 | 13.7 | 14.3 | 14.7 |
| 海流主极值条件下极值 | 有义波高Hs(m) | 6.6 | 9.2 | 10.4 | 11.6 | 12.2 |
|  | 最大波高Hmax(m) | 11.4 | 15.8 | 17.8 | 20 | 21 |
|  | 跨零周期Tz(s) | 8.9 | 10.2 | 10.7 | 11.2 | 11.5 |
|  | 谱峰周期Tp(s) | 11.5 | 13.1 | 13.7 | 14.2 | 14.5 |
|  | 平均周期Tm(s) | 11.3 | 13 | 13.6 | 14.2 | 14.5 |

海流

（来自平台所在油气田信息的海流信息）

根据平台详细设计的设计基础，平台海流的主极值与条件极值的参数如下所示。

| 海流速度（cm/s） |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 元素 |  | 回归周期 (年) |  |  |  |  |
|  |  | 1 | 10 | 25 | 50 | 100 |
| 主极值 | 表层 (0.1倍水深) | 124 | 174 | 200 | 218 | 236 |
|  | 中层 (0.5倍水深) | 100 | 140 | 161 | 175 | 192 |
|  | 底层 (0.9倍水深) | 55 | 77 | 89 | 97 | 107 |
|  | +1m@ASB | 53 | 74 | 81 | 88 | 96 |
| 风主极值条件下极值 | 表层 (0.1倍水深) | 114 | 158 | 184 | 201 | 221 |
|  | 中层 (0.5倍水深) | 92 | 124 | 141 | 151 | 163 |
|  | 底层 (0.9倍水深) | 46 | 69 | 77 | 80 | 86 |
|  | +1m@ASB | 45 | 64 | 70 | 76 | 83 |
| 波浪主极值条件下极值 | 表层 (0.1倍水深) | 117 | 158 | 182 | 199 | 217 |
|  | 中层 (0.5倍水深) | 92 | 127 | 143 | 151 | 161 |
|  | 底层 (0.9倍水深) | 48 | 69 | 76 | 79 | 83 |
|  | +1m@ASB | 45 | 62 | 68 | 74 | 81 |

风

（来自平台所在油气田信息的风信息）

根据平台详细设计的设计基础，平台风的主极值与条件极值的参数如下所示。以下风速是在海面上10m参考高程处的1小时平均风速用于静力分析中。

| 风速 @10m (m/s) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 元素 |  | 回归周期 (年) |  |  |  |  |
|  |  | 1 | 10 | 25 | 50 | 100 |
| 主极值 | 1 h | 30.3 | 35.5 | 39.1 | 41.3 | 43.4 |
|  | 10 min | 33.4 | 39.5 | 43.7 | 46.4 | 48.9 |
|  | 1 min | 37.4 | 44.6 | 49.6 | 52.9 | 55.9 |
|  | 3 s | 42.5 | 51.2 | 57.4 | 61.4 | 65.1 |
| 波浪主极值条件下极值 | 1 h | 28.4 | 33.5 | 37.2 | 39.6 | 41.8 |
|  | 10 min | 31.2 | 37.2 | 41.5 | 44.5 | 47 |
|  | 1 min | 34.9 | 42.1 | 47.1 | 50.8 | 53.9 |
|  | 3 s | 39.8 | 48.3 | 54.5 | 58.9 | 62.7 |
| 海流主极值条件下极值 | 1 h | 29 | 33.8 | 36.7 | 38.6 | 40.2 |
|  | 10 min | 32 | 37.5 | 41.1 | 43.4 | 45.2 |
|  | 1 min | 35.6 | 42.4 | 46.7 | 49.5 | 51.8 |
|  | 3 s | 40.6 | 48.7 | 54 | 57.4 | 60.3 |

海生物

海生物信息如下所示。（来自平台基本信息的海生物信息）

| 层数 |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 高度区域 | 上限(m) | 0 | -15 | -30 | -50 | -60 | -70 | -80 | -95 | -110 |
|  | 下限(m) | -15 | -30 | -50 | -60 | -70 | -80 | -95 | -110 | -122 |
| 海生物 | 厚度（cm) | 10 | 10 | 10 | 4.5 | 4.5 | 4.5 | 4 | 4 | 4 |
| 海生物密度(t/m3) |  | 1.4 |  |  |  |  |  |  |  |  |

飞溅区和腐蚀余量

海生物信息如下所示。（来自平台基本信息的飞溅区腐蚀余量）

| 飞溅区上限(m) | 飞溅区下限(m) | 腐蚀余量(mm/y) |
| --- | --- | --- |
| 8.54 | 3.75 | 7.5 |

基础冲刷

在分析中，考虑1.5m（来自平台基本信息的桩基信息基础冲刷）冲刷深度。

- 分析模型

程序和坐标系统

使用SACS 程序（由美国EDI公司开发）进行结构的总体分析、构件和节点的应力校核。

图3.1 整体模型（根据模型节点构件信息绘制）

图3.2  坐标系统（根据模型节点构件及水平层信息绘制）

结构模拟

（该部分内容内置）

甲板结构被准确的模拟为三维框架，以得到可靠的分析结果。

分析中使用三维空间框架的导管架模型。在模型中，模拟了所有的主要结构构件。所有的节点均被处理为刚性节点，每一构件的两端均为刚性连接。

在计算机模型中，立管、电缆护管、靠船构件和泵护管等附属结构被模拟为dummy构件，不考虑其刚度贡献，只考虑其重量和环境载荷。

结构材料等级和强度采用原详细设计数据。

桩基础结构模拟

（该部分内容内置）

桩土基础的模拟在设计水平分析和极限强度分析中采用P-Y，T-Z，Q-Z曲线方法。

- 设计水平分析

静力分析是平台在位分析的重要内容，该分析考虑在平台生存期内所有可能遭遇的因素的影响，这些因素主要包括环境条件、地基条件以及相关的操作要求。分析的目的是筛选在极端载荷下导管架的关键构件和节点强度要求、桩基承载力安全系数满足要求。（该部分内容内置）

环境载荷参数

静力分析分别计算极端和操作工况，操作工况采用1年1遇环境条件。对极端工况，100年重现期的风速、波高和流速作为环境荷载要素；风速采用１小时平均风速，考虑波浪的方向散射及非规则性，波浪运动系数极端工况取0.95，操作工况取1.00。

根据API RP 2A，导管架海流阻挡系数如下取值：

端面：   0.8

对角线： 0.85

（以上内容内置）

环境荷载计算8个方向，分别为 0°，46°，90°，134°，180°，226°，270°，314°。波浪理论采用STOKS V。（该内容数据和理论来自模型/海况文件，读取代码后续开发）

结构动力响应

结构动力响应参考详细设计报告。（内容内置）

基本载荷工况

基本工况描述和载荷值如下表所示。（来自结果读取的基本工况信息）

基本工况描述

| 序号 | 名称 | 描述 |
| --- | --- | --- |
| 1 | ICC1 | ICCP LOAD @236.41 DEG |
| 2 | ICC2 | ICCP LOAD @ 56.41 DEG |
| 3 | ICC3 | ICCP LOAD @ 90.00 DEG |
| 4 | ICC4 | ICCP LOAD @123.59 DEG |
| 5 | ICC5 | USER GENERATED LOADS |
| 6 | ICC6 | USER GENERATED LOADS |
| 7 | ICC7 | ICCP LOAD @303.59 DEG |
| 8 | ICC8 | ICCP LOAD @303.59 DEG |
| 9 | JKTB | USER GENERATED LOADS |
| 10 | JKTW | USER GENERATED LOADS |
| 11 | DONW | USER GENERATED LOADS |
| 12 | DONE | USER GENERATED LOADS |
| 13 | DOSW | USER GENERATED LOADS |
| 14 | DOSE | USER GENERATED LOADS |
| 15 | DENW | USER GENERATED LOADS |
| 16 | DENE | USER GENERATED LOADS |
| 17 | DESW | USER GENERATED LOADS |
| 18 | DESE | USER GENERATED LOADS |
| 19 | DKST | USER GENERATED LOADS |
| 20 | DAPP | USER GENERATED LOADS |
| 21 | LMU | USER GENERATED LOADS |
| 22 | JGOH | JACKET MODELED GRAVITY FOR OPERATING HIGH WATER LEVEL |
| 23 | JGOL | JACKET MODELED GRAVITY FOR OPERATING LOW WATER LEVEL |
| 24 | JGEH | JACKET MODELED GRAVITY FOR EXTREME HIGH WATER LEVEL |
| 25 | JGEL | JACKET MODELED GRAVITY FOR EXTREME LOW WATER LEVEL |
| 26 | W101 | PLATFORM OPERATING WIND LOAD @ 0.00 DEG (MAX WAVE) |
| 27 | W102 | PLATFORM OPERATING WIND LOAD @ 56.41 DEG (MAX WAVE) |
| 28 | W103 | PLATFORM OPERATING WIND LOAD @ 90.00 DEG (MAX WAVE) |
| 29 | W104 | PLATFORM OPERATING WIND LOAD @ 123.59 DEG (MAX WAVE) |
| 30 | W105 | PLATFORM OPERATING WIND LOAD @ 180.00 DEG (MAX WAVE) |
| 31 | W106 | PLATFORM OPERATING WIND LOAD @ 236.41 DEG (MAX WAVE) |
| 32 | W107 | PLATFORM OPERATING WIND LOAD @ 270.00 DEG (MAX WAVE) |
| 33 | W108 | PLATFORM OPERATING WIND LOAD @ 303.59 DEG (MAX WAVE) |
| 34 | W111 | PLATFORM STORM WIND LOAD @ 0.00 DEG (MAX WAVE) |
| 35 | W112 | PLATFORM STORM WIND LOAD @ 56.41 DEG (MAX WAVE) |
| 36 | W113 | PLATFORM STORM WIND LOAD @ 90.00 DEG (MAX WAVE) |
| 37 | W114 | PLATFORM STORM WIND LOAD @ 123.59 DEG (MAX WAVE) |
| 38 | W115 | PLATFORM STORM WIND LOAD @ 180.00 DEG (MAX WAVE) |
| 39 | W116 | PLATFORM STORM WIND LOAD @ 236.41 DEG (MAX WAVE) |
| 40 | W117 | PLATFORM STORM WIND LOAD @ 270.00 DEG (MAX WAVE) |
| 41 | W118 | PLATFORM STORM WIND LOAD @ 303.59 DEG (MAX WAVE) |
| 42 | OWH1 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 0.00 DEG |
| 43 | OWH2 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 56.41 DEG |
| 44 | OWH3 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 90.00 DEG |
| 45 | OWH4 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 123.59 DEG |
| 46 | OWH5 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 180.00 DEG |
| 47 | OWH6 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 236.41 DEG |
| 48 | OWH7 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 270.00 DEG |
| 49 | OWH8 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 303.59 DEG |
| 50 | OWL1 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 0.00 DEG |
| 51 | OWL2 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 56.41 DEG |
| 52 | OWL3 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 90.00 DEG |
| 53 | OWL4 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 123.59 DEG |
| 54 | OWL5 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 180.00 DEG |
| 55 | OWL6 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 236.41 DEG |
| 56 | OWL7 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 270.00 DEG |
| 57 | OWL8 | 1-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 303.59 DEG |
| 58 | EWH1 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 0.00 D |
| 59 | EWH2 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 56.41 D |
| 60 | EWH3 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 90.00 D |
| 61 | EWH4 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 123.59 D |
| 62 | EWH5 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 180.00 D |
| 63 | EWH6 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 236.41 D |
| 64 | EWH7 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 270.00 D |
| 65 | EWH8 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR HIGH WATER LEVEL@ 303.59 D |
| 66 | EWL1 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 0.00 DE |
| 67 | EWL2 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 56.41 DE |
| 68 | EWL3 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 90.00 DE |
| 69 | EWL4 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 123.59 DE |
| 70 | EWL5 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 180.00 DE |
| 71 | EWL6 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 236.41 DE |
| 72 | EWL7 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 270.00 DE |
| 73 | EWL8 | 100-YR MAX WAVE ASSOCIATED CURRENT+WAVE FOR LOW WATER LEVEL@ 303.59 DE |
| 74 | DMA | DRY WEIGHT OF MECHANICAL EQUIPMENT |
| 75 | OMA | CONTENT WEIGHT OF MECHANICAL EQUIPMENT |
| 76 | DPI | DRY WEIGHT OF PIPING |
| 77 | OPI | CONTENT WEIGHT OF PIPING |
| 78 | EL | DRY WEIGHT OF ELECTRICAL EQUIPMENT+CABLE+CABLE TRAY AND ETC |
| 79 | IN | DRY WEIGHT OF INSTRUMENT EQUIPMENT |
| 80 | CO | DRY WEIGHT OF COMMUNICATION EQUIPMENT |
| 81 | OU | DRY WEIGHT OF OUTFITTING |
| 82 | SA | DRY WEIGHT OF SAFETY EQUIPMENT |
| 83 | DFLA | WEIGHT OF FLARE BOOM |
| 84 | HVAC | DRY WEIGHT OF HVAC EQUIPMENT |
| 85 | CC | DRY WEIGHT OF CORROSION CONTROL |
| 86 | ALIV | ANTI-LIVE LOAD |
| 87 | LIV | LIVE LOAD |
| 88 | DLQ | DRY WEIGHT OF LIVIN QUARTER |
| 89 | OLQ | CONTENT WEIGHT OF LIVING QUARTER |
| 90 | LLQ | LIVE LOAD OF LIVING QUARTER |
| 91 | DADE | DRY WEIGHT OF EQUIPMENTS MDR |
| 92 | OADE | WET WEIGHT OF EQUIPMENTS MDR |
| 93 | DDSM | DRY WEIGHT OF DSM |
| 94 | ODSM | CONTENT WEIGHT OF DSM |
| 95 | LDSM | LIVE LOAD OF DSM |

基本工况载荷值

| 工况 名称 | FX （kN） | FY （kN） | FZ （kN） | MX （kN.m） | MY （kN.m） | MZ （kN.m） | 固定载荷 （kN） | 海生物方法浮力 （kN） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ICC1 | 434.16 | 0 | -34.6 | 0.1 | 46170.6 | 1.4 | 0 | 0 |
| ICC2 | 240.2 | 361.66 | -34.6 | -38482.6 | 25532.2 | -287.7 | 0 | 0 |
| ICC3 | 0 | 434.16 | -34.6 | -46196.5 | -26 | -346.4 | 0 | 0 |
| ICC4 | -240.2 | 361.66 | -34.6 | -38482.6 | -25584.2 | -289.3 | 0 | 0 |
| ICC5 | -434.16 | 0 | -34.6 | 0.1 | -46222.6 | -1.4 | 0 | 0 |
| ICC6 | -240.2 | -361.66 | -34.6 | 38482.9 | -25584.2 | 287.7 | 0 | 0 |
| ICC7 | 0 | -434.16 | -34.6 | 46196.8 | -26 | 346.4 | 0 | 0 |
| ICC8 | 240.2 | -361.66 | -34.6 | 38482.9 | 25532.2 | 289.3 | 0 | 0 |
| JKTB | 0 | 0 | 5873.08 | -82347.7 | -4024.2 | 0 | 0 | 0 |
| JKTW | 0 | 0 | -14817.1 | 99953 | 28802.6 | 0 | 0 | 0 |
| DONW | 0 | 0 | -22604.1 | -153673 | 641657.6 | 0 | 0 | 0 |
| DONE | 0 | 0 | -22601.4 | -153668 | 788634.9 | 0 | 0 | 0 |
| DOSW | 0 | 0 | -22604.2 | 176948.8 | 641660.1 | 0 | 0 | 0 |
| DOSE | 0 | 0 | -22600.8 | 176909.9 | 788612.1 | 0 | 0 | 0 |
| DENW | 0 | 0 | -22226.4 | -163892 | 613352.6 | 0 | 0 | 0 |
| DENE | 0 | 0 | -22225.6 | -173436 | 763500.3 | 0 | 0 | 0 |
| DESW | 0 | 0 | -22226.3 | 186972.7 | 613350.2 | 0 | 0 | 0 |
| DESE | 0 | 0 | -22225.8 | 186966.9 | 792430.5 | 0 | 0 | 0 |
| DKST | 0 | 0 | -62437.4 | 8442.3 | 90798.5 | 0 | 0 | 0 |
| DAPP | 0 | 0 | -8834.87 | -13082.3 | -100621 | 0 | 0 | 0 |
| LMU | 0 | 0 | -4316.54 | 0 | 0 | 0 | 0 | 0 |
| JGOH | 0 | 0 | -55396.6 | 56795.4 | 986073.2 | 0 | 216237.55 | 160836.91 |
| JGOL | 0 | 0 | -57654.8 | 56733.3 | 989984.5 | 0 | 216237.55 | 158578.7 |
| JGEH | 0 | 0 | -55224.1 | 56799.6 | 985936.8 | 0 | 216237.55 | 161009.41 |
| JGEL | 0 | 0 | -57988.6 | 56722.8 | 990818.8 | 0 | 216237.55 | 158244.89 |
| W101 | 1903.96 | 0.3 | -1.45 | -17.8 | 296920.4 | 19 | 0 | 0 |
| W102 | 1053.61 | 3566.39 | -0.23 | -566657 | 164306.7 | -957.6 | 0 | 0 |
| W103 | 0.3 | 4281.09 | 0.68 | -680234 | 43.9 | -1162.2 | 0 | 0 |
| W104 | -1053.11 | 3566.06 | 1.37 | -566637 | -164234 | -978.6 | 0 | 0 |
| W105 | -1903.96 | -0.3 | 1.45 | 17.8 | -296920 | -19 | 0 | 0 |
| W106 | -1053.61 | -3566.39 | 0.23 | 566657.2 | -164307 | 957.6 | 0 | 0 |
| W107 | -0.3 | -4281.09 | -0.68 | 680234.1 | -43.9 | 1162.2 | 0 | 0 |
| W108 | 1053.11 | -3566.06 | -1.37 | 566637.3 | 164233.5 | 978.6 | 0 | 0 |
| W111 | 4761.77 | 0.73 | -3.89 | -38.2 | 744665.8 | 55.6 | 0 | 0 |
| W112 | 2635.04 | 8986.43 | -0.81 | -1430451 | 412071.9 | -2787.3 | 0 | 0 |
| W113 | 0.73 | 10787.32 | 1.61 | -1717167 | 106.1 | -3383 | 0 | 0 |
| W114 | -2633.83 | 8985.62 | 3.49 | -1430409 | -411895 | -2848.8 | 0 | 0 |
| W115 | -4761.77 | -0.73 | 3.89 | 38.2 | -744666 | -55.6 | 0 | 0 |
| W116 | -2635.04 | -8986.43 | 0.81 | 1430452 | -412072 | 2787.3 | 0 | 0 |
| W117 | -0.73 | -10787.3 | -1.61 | 1717167 | -106.1 | 3383 | 0 | 0 |
| W118 | 2633.83 | -8985.62 | -3.49 | 1430409 | 411895 | 2848.8 | 0 | 0 |
| OWH1 | 18305.11 | 7.02 | -737.05 | 1092.6 | 1618334 | 14907.7 | 0 | 0 |
| OWH2 | 14670.77 | 22882 | -1805.86 | -2239147 | 1368456 | 191722.2 | 0 | 0 |
| OWH3 | -24.75 | 28077.12 | -1479.31 | -2792791 | -372.3 | 175380.7 | 0 | 0 |
| OWH4 | -15545.4 | 24217.61 | -800.99 | -2367588 | -1447551 | 239264.8 | 0 | 0 |
| OWH5 | -21232.1 | 5.04 | 25.48 | -488.9 | -1935340 | -12097.1 | 0 | 0 |
| OWH6 | -15554.1 | -24311.2 | -776.27 | 2375869 | -1446705 | -252397 | 0 | 0 |
| OWH7 | -31.52 | -28207.9 | -1516.19 | 2804261 | 812.1 | -176006 | 0 | 0 |
| OWH8 | 14727.15 | -22928.7 | -1797.11 | 2243508 | 1372231 | -179510 | 0 | 0 |
| OWL1 | 18832.4 | 10.3 | -609.09 | 780.9 | 1631995 | 16374.3 | 0 | 0 |
| OWL2 | 15172.38 | 23683.25 | -1705.62 | -2280234 | 1394883 | 199716.3 | 0 | 0 |
| OWL3 | -4.04 | 28747.92 | -1314.44 | -2803357 | 2090.6 | 180339.9 | 0 | 0 |
| OWL4 | -15966.4 | 24808.53 | -576.97 | -2378124 | -1463974 | 245001.7 | 0 | 0 |
| OWL5 | -21525.9 | 11.12 | 365.96 | -1280.3 | -1916234 | -13152 | 0 | 0 |
| OWL6 | -15960.1 | -24909 | -528.26 | 2386882 | -1461365 | -259415 | 0 | 0 |
| OWL7 | -11.58 | -28898.1 | -1326.39 | 2816763 | 3329.6 | -180748 | 0 | 0 |
| OWL8 | 15237.75 | -23738 | -1682.85 | 2285464 | 1399582 | -186075 | 0 | 0 |
| EWH1 | 74063.24 | 31.8 | -2469.42 | 618.7 | 6356806 | 65919.9 | 0 | 0 |
| EWH2 | 51834.77 | 78935.74 | -2453.81 | -7237856 | 4551812 | 687171.4 | 0 | 0 |
| EWH3 | 20.98 | 89979.17 | -1118.64 | -8299596 | 2009.9 | 596594.8 | 0 | 0 |
| EWH4 | -52741 | 80145.68 | 431.25 | -7329748 | -4621683 | 694848.2 | 0 | 0 |
| EWH5 | -76407.2 | 42.35 | -605.4 | -4153.4 | -6567232 | -63656.9 | 0 | 0 |
| EWH6 | -52695.9 | -80284.8 | 343.06 | 7339494 | -4615825 | -759684 | 0 | 0 |
| EWH7 | -24.29 | -90154.2 | -1354.63 | 8313326 | 3741.3 | -597233 | 0 | 0 |
| EWH8 | 51900.88 | -78949.5 | -2562.5 | 7239607 | 4559040 | -622079 | 0 | 0 |
| EWL1 | 77285.91 | 24.73 | -2918.91 | 1831.1 | 6534164 | 71808.2 | 0 | 0 |
| EWL2 | 54292.34 | 82762.85 | -2462.25 | -7500080 | 4699840 | 713316.6 | 0 | 0 |
| EWL3 | 20.84 | 94453.3 | -984.07 | -8609628 | 1450 | 619306.9 | 0 | 0 |
| EWL4 | -55089.7 | 83815.58 | -1468.19 | -7592115 | -4772365 | 681660.6 | 0 | 0 |
| EWL5 | -79480.6 | 35.71 | 269.3 | -4238 | -6712058 | -67635.4 | 0 | 0 |
| EWL6 | -55099.9 | -83946.5 | -1563.01 | 7602216 | -4768714 | -752582 | 0 | 0 |
| EWL7 | -22.85 | -94636.1 | -1234.83 | 8623363 | 4025.9 | -620000 | 0 | 0 |
| EWL8 | 54371.5 | -82786.8 | -2572.67 | 7503059 | 4708807 | -642222 | 0 | 0 |
| DMA | 0 | 0 | -34762.2 | 8660.6 | 37380.2 | 0 | 0 | 0 |
| OMA | 0 | 0 | -18369.1 | 29048.4 | 14076.9 | 0 | 0 | 0 |
| DPI | 0 | 0 | -15782.8 | -5847.7 | 56596.4 | 0 | 0 | 0 |
| OPI | 0 | 0 | -5520.03 | -1431.3 | 26584 | 0 | 0 | 0 |
| EL | 0 | 0 | -6470.28 | -1025.6 | -165038 | 0 | 0 | 0 |
| IN | 0 | 0 | -2174.39 | 4058.1 | 17224.8 | 0 | 0 | 0 |
| CO | 0 | 0 | -113.31 | -401.7 | -1328.8 | 0 | 0 | 0 |
| OU | 0 | 0 | -3862.84 | -429.5 | -105624 | 0 | 0 | 0 |
| SA | 0 | 0 | -415.67 | 1706.7 | -8307.8 | 0 | 0 | 0 |
| DFLA | 0 | 0 | -2236.81 | 30196.9 | 171504.5 | 0 | 0 | 0 |
| HVAC | 0 | 0 | -433.26 | -469.4 | -16069.5 | 0 | 0 | 0 |
| CC | 0 | 0 | -355.47 | -53.3 | -554.5 | 0 | 0 | 0 |
| ALIV | 0 | 0 | -13681.1 | -60.6 | 12013.1 | 0 | 0 | 0 |
| LIV | 0 | 0 | -28222.3 | -2539.9 | 9144.7 | 0 | 0 | 0 |
| DLQ | 0 | 0 | -12971.5 | 609.7 | -592927 | 0 | 0 | 0 |
| OLQ | 0 | 0 | -345.33 | 2136.6 | -12898.2 | 0 | 0 | 0 |
| LLQ | 0 | 0 | -5640.75 | 1162 | -256598 | 0 | 0 | 0 |
| DADE | 0 | 0 | -1896.97 | 2277.8 | 59303.6 | 0 | 0 | 0 |
| OADE | 0 | 0 | -789.87 | -1018.8 | 25053.3 | 0 | 0 | 0 |
| DDSM | 0 | 0 | -16429.4 | -17730.5 | 81964.1 | 0 | 0 | 0 |
| ODSM | 0 | 0 | -19421.4 | -5224.1 | 172045 | 0 | 0 | 0 |
| LDSM | 0 | 0 | -3959.91 | 7523.8 | 25486 | 0 | 0 | 0 |

组合载荷工况

组合工况描述和载荷值如下表所示。（来自结果读取的组合工况信息）

组合载荷工况描述

| 序号 | 名称 | 类别 | 描述 |
| --- | --- | --- | --- |
| 96 | DTP |  | DMA*1.000+DPI*1.000+DADE*1.000+DAPP*1.000+DFLA*1.000+ DKST*1.000+EL*1.000+IN*1.000+CO*1.000+OU*1.000+SA*1.000+ HVAC*1.000+CC*1.000+LMU*1.000 |
| 97 | OTP |  | OMA*1.000+OPI*1.000+OADE*1.000 |
| 98 | LIVE |  | LIV*1.000+ALIV*1.000 |
| 99 | WOP1 |  | DTP*1.000+OTP*1.000+LIVE*0.500+DLQ*1.000+OLQ*1.000+LLQ*0.500+ DDSM*1.000+ODSM*1.000+LDSM*0.500+DONW*1.000 |
| 100 | WOP2 |  | DTP*1.000+OTP*1.000+LIVE*0.500+DLQ*1.000+OLQ*1.000+LLQ*0.500+ DDSM*1.000+ODSM*1.000+LDSM*0.500+DONE*1.000 |
| 101 | WOP3 |  | DTP*1.000+OTP*1.000+LIVE*0.500+DLQ*1.000+OLQ*1.000+LLQ*0.500+ DDSM*1.000+ODSM*1.000+LDSM*0.500+DOSW*1.000 |
| 102 | WOP4 |  | DTP*1.000+OTP*1.000+LIVE*0.500+DLQ*1.000+OLQ*1.000+LLQ*0.500+ DDSM*1.000+ODSM*1.000+LDSM*0.500+DOSE*1.000 |
| 103 | WEX1 |  | DTP*1.000+OTP*0.750+LIVE*0.375+DLQ*1.000+OLQ*0.750+LLQ*0.375+ DDSM*1.000+ODSM*0.750+LDSM*0.375+DENW*1.000 |
| 104 | WEX2 |  | DTP*1.000+OTP*0.750+LIVE*0.375+DLQ*1.000+OLQ*0.750+LLQ*0.375+ DDSM*1.000+ODSM*0.750+LDSM*0.375+DENE*1.000 |
| 105 | WEX3 |  | DTP*1.000+OTP*0.750+LIVE*0.375+DLQ*1.000+OLQ*0.750+LLQ*0.375+ DDSM*1.000+ODSM*0.750+LDSM*0.375+DESW*1.000 |
| 106 | WEX4 |  | DTP*1.000+OTP*0.750+LIVE*0.375+DLQ*1.000+OLQ*0.750+LLQ*0.375+ DDSM*1.000+ODSM*0.750+LDSM*0.375+DESE*1.000 |
| 107 | WLT1 |  | DTP*1.000+OTP*0.250+DLQ*1.000+OLQ*0.250+DDSM*1.000+ODSM*0.250+ DENW*1.000 |
| 108 | OPNW |  | WOP1*1.000+JKTW*1.050+JKTB*1.000 |
| 109 | OPNE |  | WOP2*1.000+JKTW*1.050+JKTB*1.000 |
| 110 | OPSW |  | WOP3*1.000+JKTW*1.050+JKTB*1.000 |
| 111 | OPSE |  | WOP4*1.000+JKTW*1.050+JKTB*1.000 |
| 112 | EXNW |  | WEX1*1.000+JKTW*1.050+JKTB*1.000 |
| 113 | EXNE |  | WEX2*1.000+JKTW*1.050+JKTB*1.000 |
| 114 | EXSW |  | WEX3*1.000+JKTW*1.050+JKTB*1.000 |
| 115 | EXSE |  | WEX4*1.000+JKTW*1.050+JKTB*1.000 |
| 116 | LTNW |  | WLT1*1.000+JKTW*1.050+JKTB*1.000 |
| 117 | OH11 | Operation | JGOH*1.000+W101*1.000+OWH1*1.124+OPNW*1.000+ICC1*1.000 |
| 118 | OH12 | Operation | JGOH*1.000+W102*1.000+OWH2*1.039+OPNW*1.000+ICC2*1.000 |
| 119 | OH13 | Operation | JGOH*1.000+W103*1.000+OWH3*1.087+OPNW*1.000+ICC3*1.000 |
| 120 | OH14 | Operation | JGOH*1.000+W104*1.000+OWH4*1.096+OPNW*1.000+ICC4*1.000 |
| 121 | OH15 | Operation | JGOH*1.000+W105*1.000+OWH5*1.136+OPNW*1.000+ICC5*1.000 |
| 122 | OH16 | Operation | JGOH*1.000+W106*1.000+OWH6*1.100+OPNW*1.000+ICC6*1.000 |
| 123 | OH17 | Operation | JGOH*1.000+W107*1.000+OWH7*1.089+OPNW*1.000+ICC7*1.000 |
| 124 | OH18 | Operation | JGOH*1.000+W108*1.000+OWH8*1.042+OPNW*1.000+ICC8*1.000 |
| 125 | OL11 | Operation | JGOL*1.000+W101*1.000+OWL1*1.124+OPNW*1.000+ICC1*1.000 |
| 126 | OL12 | Operation | JGOL*1.000+W102*1.000+OWL2*1.039+OPNW*1.000+ICC2*1.000 |
| 127 | OL13 | Operation | JGOL*1.000+W103*1.000+OWL3*1.087+OPNW*1.000+ICC3*1.000 |
| 128 | OL14 | Operation | JGOL*1.000+W104*1.000+OWL4*1.096+OPNW*1.000+ICC4*1.000 |
| 129 | OL15 | Operation | JGOL*1.000+W105*1.000+OWL5*1.136+OPNW*1.000+ICC5*1.000 |
| 130 | OL16 | Operation | JGOL*1.000+W106*1.000+OWL6*1.100+OPNW*1.000+ICC6*1.000 |
| 131 | OL17 | Operation | JGOL*1.000+W107*1.000+OWL7*1.089+OPNW*1.000+ICC7*1.000 |
| 132 | OL18 | Operation | JGOL*1.000+W108*1.000+OWL8*1.042+OPNW*1.000+ICC8*1.000 |
| 133 | EH11 | Extreme | JGEH*1.000+W111*1.000+EWH1*1.101+EXNW*1.000+ICC1*1.000 |
| 134 | EH12 | Extreme | JGEH*1.000+W112*1.000+EWH2*1.058+EXNW*1.000+ICC2*1.000 |
| 135 | EH13 | Extreme | JGEH*1.000+W113*1.000+EWH3*1.114+EXNW*1.000+ICC3*1.000 |
| 136 | EH14 | Extreme | JGEH*1.000+W114*1.000+EWH4*1.091+EXNW*1.000+ICC4*1.000 |
| 137 | EH15 | Extreme | JGEH*1.000+W115*1.000+EWH5*1.121+EXNW*1.000+ICC5*1.000 |
| 138 | EH16 | Extreme | JGEH*1.000+W116*1.000+EWH6*1.079+EXNW*1.000+ICC6*1.000 |
| 139 | EH17 | Extreme | JGEH*1.000+W117*1.000+EWH7*1.110+EXNW*1.000+ICC7*1.000 |
| 140 | EH18 | Extreme | JGEH*1.000+W118*1.000+EWH7*1.062+EXNW*1.000+ICC8*1.000 |
| 141 | EL11 | Extreme | JGEL*1.000+W111*1.000+EWL1*1.101+EXNW*1.000+ICC1*1.000 |
| 142 | EL12 | Extreme | JGEL*1.000+W112*1.000+EWL2*1.058+EXNW*1.000+ICC2*1.000 |
| 143 | EL13 | Extreme | JGEL*1.000+W113*1.000+EWL3*1.114+EXNW*1.000+ICC3*1.000 |
| 144 | EL14 | Extreme | JGEL*1.000+W114*1.000+EWL4*1.091+EXNW*1.000+ICC4*1.000 |
| 145 | EL15 | Extreme | JGEL*1.000+W115*1.000+EWL5*1.121+EXNW*1.000+ICC5*1.000 |
| 146 | EL16 | Extreme | JGEL*1.000+W116*1.000+EWL6*1.079+EXNW*1.000+ICC6*1.000 |
| 147 | EL17 | Extreme | JGEL*1.000+W117*1.000+EWL7*1.110+EXNW*1.000+ICC7*1.000 |
| 148 | EL18 | Extreme | JGEL*1.000+W118*1.000+EWL7*1.062+EXNW*1.000+ICC8*1.000 |
| 149 | OH21 | Operation | JGOH*1.000+W101*1.000+OWH1*1.124+OPNE*1.000+ICC1*1.000 |
| 150 | OH22 | Operation | JGOH*1.000+W102*1.000+OWH2*1.039+OPNE*1.000+ICC2*1.000 |
| 151 | OH23 | Operation | JGOH*1.000+W103*1.000+OWH3*1.087+OPNE*1.000+ICC3*1.000 |
| 152 | OH24 | Operation | JGOH*1.000+W104*1.000+OWH4*1.096+OPNE*1.000+ICC4*1.000 |
| 153 | OH25 | Operation | JGOH*1.000+W105*1.000+OWH5*1.136+OPNE*1.000+ICC5*1.000 |
| 154 | OH26 | Operation | JGOH*1.000+W106*1.000+OWH6*1.100+OPNE*1.000+ICC6*1.000 |
| 155 | OH27 | Operation | JGOH*1.000+W107*1.000+OWH7*1.089+OPNE*1.000+ICC7*1.000 |
| 156 | OH28 | Operation | JGOH*1.000+W108*1.000+OWH8*1.042+OPNE*1.000+ICC8*1.000 |
| 157 | OL21 | Operation | JGOL*1.000+W101*1.000+OWL1*1.124+OPNE*1.000+ICC1*1.000 |
| 158 | OL22 | Operation | JGOL*1.000+W102*1.000+OWL2*1.039+OPNE*1.000+ICC2*1.000 |
| 159 | OL23 | Operation | JGOL*1.000+W103*1.000+OWL3*1.087+OPNE*1.000+ICC3*1.000 |
| 160 | OL24 | Operation | JGOL*1.000+W104*1.000+OWL4*1.096+OPNE*1.000+ICC4*1.000 |
| 161 | OL25 | Operation | JGOL*1.000+W105*1.000+OWL5*1.136+OPNE*1.000+ICC5*1.000 |
| 162 | OL26 | Operation | JGOL*1.000+W106*1.000+OWL6*1.100+OPNE*1.000+ICC6*1.000 |
| 163 | OL27 | Operation | JGOL*1.000+W107*1.000+OWL7*1.089+OPNE*1.000+ICC7*1.000 |
| 164 | OL28 | Operation | JGOL*1.000+W108*1.000+OWL8*1.042+OPNE*1.000+ICC8*1.000 |
| 165 | EH21 | Extreme | JGEH*1.000+W111*1.000+EWH1*1.101+EXNE*1.000+ICC1*1.000 |
| 166 | EH22 | Extreme | JGEH*1.000+W112*1.000+EWH2*1.058+EXNE*1.000+ICC2*1.000 |
| 167 | EH23 | Extreme | JGEH*1.000+W113*1.000+EWH3*1.114+EXNE*1.000+ICC3*1.000 |
| 168 | EH24 | Extreme | JGEH*1.000+W114*1.000+EWH4*1.091+EXNE*1.000+ICC4*1.000 |
| 169 | EH25 | Extreme | JGEH*1.000+W115*1.000+EWH5*1.121+EXNE*1.000+ICC5*1.000 |
| 170 | EH26 | Extreme | JGEH*1.000+W116*1.000+EWH6*1.079+EXNE*1.000+ICC6*1.000 |
| 171 | EH27 | Extreme | JGEH*1.000+W117*1.000+EWH7*1.110+EXNE*1.000+ICC7*1.000 |
| 172 | EH28 | Extreme | JGEH*1.000+W118*1.000+EWH8*1.062+EXNE*1.000+ICC8*1.000 |
| 173 | EL21 | Extreme | JGEL*1.000+W111*1.000+EWL1*1.101+EXNE*1.000+ICC1*1.000 |
| 174 | EL22 | Extreme | JGEL*1.000+W112*1.000+EWL2*1.058+EXNE*1.000+ICC2*1.000 |
| 175 | EL23 | Extreme | JGEL*1.000+W113*1.000+EWL3*1.114+EXNE*1.000+ICC3*1.000 |
| 176 | EL24 | Extreme | JGEL*1.000+W114*1.000+EWL4*1.091+EXNE*1.000+ICC4*1.000 |
| 177 | EL25 | Extreme | JGEL*1.000+W115*1.000+EWL5*1.121+EXNE*1.000+ICC5*1.000 |
| 178 | EL26 | Extreme | JGEL*1.000+W116*1.000+EWL6*1.079+EXNE*1.000+ICC6*1.000 |
| 179 | EL27 | Extreme | JGEL*1.000+W117*1.000+EWL7*1.110+EXNE*1.000+ICC7*1.000 |
| 180 | EL28 | Extreme | JGEL*1.000+W118*1.000+EWL8*1.062+EXNE*1.000+ICC8*1.000 |
| 181 | OH31 | Operation | JGOH*1.000+W101*1.000+OWH1*1.124+OPSW*1.000+ICC1*1.000 |
| 182 | OH32 | Operation | JGOH*1.000+W102*1.000+OWH2*1.039+OPSW*1.000+ICC2*1.000 |
| 183 | OH33 | Operation | JGOH*1.000+W103*1.000+OWH3*1.087+OPSW*1.000+ICC3*1.000 |
| 184 | OH34 | Operation | JGOH*1.000+W104*1.000+OWH4*1.096+OPSW*1.000+ICC4*1.000 |
| 185 | OH35 | Operation | JGOH*1.000+W105*1.000+OWH5*1.136+OPSW*1.000+ICC5*1.000 |
| 186 | OH36 | Operation | JGOH*1.000+W106*1.000+OWH6*1.100+OPSW*1.000+ICC6*1.000 |
| 187 | OH37 | Operation | JGOH*1.000+W107*1.000+OWH7*1.089+OPSW*1.000+ICC7*1.000 |
| 188 | OH38 | Operation | JGOH*1.000+W108*1.000+OWH8*1.042+OPSW*1.000+ICC8*1.000 |
| 189 | OL31 | Operation | JGOL*1.000+W101*1.000+OWL1*1.124+OPSW*1.000+ICC1*1.000 |
| 190 | OL32 | Operation | JGOL*1.000+W102*1.000+OWL2*1.039+OPSW*1.000+ICC2*1.000 |
| 191 | OL33 | Operation | JGOL*1.000+W103*1.000+OWL3*1.087+OPSW*1.000+ICC3*1.000 |
| 192 | OL34 | Operation | JGOL*1.000+W104*1.000+OWL4*1.096+OPSW*1.000+ICC4*1.000 |
| 193 | OL35 | Operation | JGOL*1.000+W105*1.000+OWL5*1.136+OPSW*1.000+ICC5*1.000 |
| 194 | OL36 | Operation | JGOL*1.000+W106*1.000+OWL6*1.100+OPSW*1.000+ICC6*1.000 |
| 195 | OL37 | Operation | JGOL*1.000+W107*1.000+OWL7*1.089+OPSW*1.000+ICC7*1.000 |
| 196 | OL38 | Operation | JGOL*1.000+W108*1.000+OWL8*1.042+OPSW*1.000+ICC8*1.000 |
| 197 | EH31 | Extreme | JGEH*1.000+W111*1.000+EWH1*1.101+EXSW*1.000+ICC1*1.000 |
| 198 | EH32 | Extreme | JGEH*1.000+W112*1.000+EWH2*1.058+EXSW*1.000+ICC2*1.000 |
| 199 | EH33 | Extreme | JGEH*1.000+W113*1.000+EWH3*1.114+EXSW*1.000+ICC3*1.000 |
| 200 | EH34 | Extreme | JGEH*1.000+W114*1.000+EWH4*1.091+EXSW*1.000+ICC4*1.000 |
| 201 | EH35 | Extreme | JGEH*1.000+W115*1.000+EWH5*1.121+EXSW*1.000+ICC5*1.000 |
| 202 | EH36 | Extreme | JGEH*1.000+W116*1.000+EWH6*1.079+EXSW*1.000+ICC6*1.000 |
| 203 | EH37 | Extreme | JGEH*1.000+W117*1.000+EWH7*1.110+EXSW*1.000+ICC7*1.000 |
| 204 | EH38 | Extreme | JGEH*1.000+W118*1.000+EWH8*1.062+EXSW*1.000+ICC8*1.000 |
| 205 | EL31 | Extreme | JGEL*1.000+W111*1.000+EWL1*1.101+EXSW*1.000+ICC1*1.000 |
| 206 | EL32 | Extreme | JGEL*1.000+W112*1.000+EWL2*1.058+EXSW*1.000+ICC2*1.000 |
| 207 | EL33 | Extreme | JGEL*1.000+W113*1.000+EWL3*1.114+EXSW*1.000+ICC3*1.000 |
| 208 | EL34 | Extreme | JGEL*1.000+W114*1.000+EWL4*1.091+EXSW*1.000+ICC4*1.000 |
| 209 | EL35 | Extreme | JGEL*1.000+W115*1.000+EWL5*1.121+EXSW*1.000+ICC5*1.000 |
| 210 | EL36 | Extreme | JGEL*1.000+W116*1.000+EWL6*1.079+EXSW*1.000+ICC6*1.000 |
| 211 | EL37 | Extreme | JGEL*1.000+W117*1.000+EWL7*1.110+EXSW*1.000+ICC7*1.000 |
| 212 | EL38 | Extreme | JGEL*1.000+W118*1.000+EWL8*1.062+EXSW*1.000+ICC8*1.000 |
| 213 | OH41 | Operation | JGOH*1.000+W101*1.000+OWH1*1.124+OPSE*1.000+ICC1*1.000 |
| 214 | OH42 | Operation | JGOH*1.000+W102*1.000+OWH2*1.039+OPSE*1.000+ICC2*1.000 |
| 215 | OH43 | Operation | JGOH*1.000+W103*1.000+OWH3*1.087+OPSE*1.000+ICC3*1.000 |
| 216 | OH44 | Operation | JGOH*1.000+W104*1.000+OWH4*1.096+OPSE*1.000+ICC4*1.000 |
| 217 | OH45 | Operation | JGOH*1.000+W105*1.000+OWH5*1.136+OPSE*1.000+ICC5*1.000 |
| 218 | OH46 | Operation | JGOH*1.000+W106*1.000+OWH6*1.100+OPSE*1.000+ICC6*1.000 |
| 219 | OH47 | Operation | JGOH*1.000+W107*1.000+OWH7*1.089+OPSE*1.000+ICC7*1.000 |
| 220 | OH48 | Operation | JGOH*1.000+W108*1.000+OWH8*1.042+OPSE*1.000+ICC8*1.000 |
| 221 | OL41 | Operation | JGOL*1.000+W101*1.000+OWL1*1.124+OPSE*1.000+ICC1*1.000 |
| 222 | OL42 | Operation | JGOL*1.000+W102*1.000+OWL2*1.039+OPSE*1.000+ICC2*1.000 |
| 223 | OL43 | Operation | JGOL*1.000+W103*1.000+OWL3*1.087+OPSE*1.000+ICC3*1.000 |
| 224 | OL44 | Operation | JGOL*1.000+W104*1.000+OWL4*1.096+OPSE*1.000+ICC4*1.000 |
| 225 | OL45 | Operation | JGOL*1.000+W105*1.000+OWL5*1.136+OPSE*1.000+ICC5*1.000 |
| 226 | OL46 | Operation | JGOL*1.000+W106*1.000+OWL6*1.100+OPSE*1.000+ICC6*1.000 |
| 227 | OL47 | Operation | JGOL*1.000+W107*1.000+OWL7*1.089+OPSE*1.000+ICC7*1.000 |
| 228 | OL48 | Operation | JGOL*1.000+W108*1.000+OWL8*1.042+OPSE*1.000+ICC8*1.000 |
| 229 | EH41 | Extreme | JGEH*1.000+W111*1.000+EWH1*1.101+EXSE*1.000+ICC1*1.000 |
| 230 | EH42 | Extreme | JGEH*1.000+W112*1.000+EWH2*1.058+EXSE*1.000+ICC2*1.000 |
| 231 | EH43 | Extreme | JGEH*1.000+W113*1.000+EWH3*1.114+EXSE*1.000+ICC3*1.000 |
| 232 | EH44 | Extreme | JGEH*1.000+W114*1.000+EWH4*1.091+EXSE*1.000+ICC4*1.000 |
| 233 | EH45 | Extreme | JGEH*1.000+W115*1.000+EWH5*1.121+EXSE*1.000+ICC5*1.000 |
| 234 | EH46 | Extreme | JGEH*1.000+W116*1.000+EWH6*1.079+EXSE*1.000+ICC6*1.000 |
| 235 | EH47 | Extreme | JGEH*1.000+W117*1.000+EWH7*1.110+EXSE*1.000+ICC7*1.000 |
| 236 | EH48 | Extreme | JGEH*1.000+W118*1.000+EWH8*1.062+EXSE*1.000+ICC8*1.000 |
| 237 | EL41 | Extreme | JGEL*1.000+W111*1.000+EWL1*1.101+EXSE*1.000+ICC1*1.000 |
| 238 | EL42 | Extreme | JGEL*1.000+W112*1.000+EWL2*1.058+EXSE*1.000+ICC2*1.000 |
| 239 | EL43 | Extreme | JGEL*1.000+W113*1.000+EWL3*1.114+EXSE*1.000+ICC3*1.000 |
| 240 | EL44 | Extreme | JGEL*1.000+W114*1.000+EWL4*1.091+EXSE*1.000+ICC4*1.000 |
| 241 | EL45 | Extreme | JGEL*1.000+W115*1.000+EWL5*1.121+EXSE*1.000+ICC5*1.000 |
| 242 | EL46 | Extreme | JGEL*1.000+W116*1.000+EWL6*1.079+EXSE*1.000+ICC6*1.000 |
| 243 | EL47 | Extreme | JGEL*1.000+W117*1.000+EWL7*1.110+EXSE*1.000+ICC7*1.000 |
| 244 | EL48 | Extreme | JGEL*1.000+W118*1.000+EWL8*1.062+EXSE*1.000+ICC8*1.000 |
| 245 | LH11 | Extreme | JGEH*1.000+W111*1.000+EWH1*1.101+LTNW*1.000+ICC1*1.000 |
| 246 | LH12 | Extreme | JGEH*1.000+W112*1.000+EWH2*1.058+LTNW*1.000+ICC2*1.000 |
| 247 | LH13 | Extreme | JGEH*1.000+W113*1.000+EWH3*1.114+LTNW*1.000+ICC3*1.000 |
| 248 | LH14 | Extreme | JGEH*1.000+W114*1.000+EWH4*1.091+LTNW*1.000+ICC4*1.000 |
| 249 | LH15 | Extreme | JGEH*1.000+W115*1.000+EWH5*1.121+LTNW*1.000+ICC5*1.000 |
| 250 | LH16 | Extreme | JGEH*1.000+W116*1.000+EWH6*1.079+LTNW*1.000+ICC6*1.000 |
| 251 | LH17 | Extreme | JGEH*1.000+W117*1.000+EWH7*1.110+LTNW*1.000+ICC7*1.000 |
| 252 | LH18 | Extreme | JGEH*1.000+W118*1.000+EWH8*1.062+LTNW*1.000+ICC8*1.000 |
| 253 | LL11 | Extreme | JGEL*1.000+W111*1.000+EWL1*1.101+LTNW*1.000+ICC1*1.000 |
| 254 | LL12 | Extreme | JGEL*1.000+W112*1.000+EWL2*1.058+LTNW*1.000+ICC2*1.000 |
| 255 | LL13 | Extreme | JGEL*1.000+W113*1.000+EWL3*1.114+LTNW*1.000+ICC3*1.000 |
| 256 | LL14 | Extreme | JGEL*1.000+W114*1.000+EWL4*1.091+LTNW*1.000+ICC4*1.000 |
| 257 | LL15 | Extreme | JGEL*1.000+W115*1.000+EWL5*1.121+LTNW*1.000+ICC5*1.000 |
| 258 | LL16 | Extreme | JGEL*1.000+W116*1.000+EWL6*1.079+LTNW*1.000+ICC6*1.000 |
| 259 | LL17 | Extreme | JGEL*1.000+W117*1.000+EWL7*1.110+LTNW*1.000+ICC7*1.000 |
| 260 | LL18 | Extreme | JGEL*1.000+W118*1.000+EWL8*1.062+LTNW*1.000+ICC8*1.000 |

组合工况载荷值

| 序号 | 工况 名称 | FX （kN） | FY （kN） | FZ （kN） | MX （kN.m） | MY （kN.m） | MZ （kN.m） |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 96 | DTP | 0 | 0 | -144093 | 34032.8 | 35264.7 | 0 |
| 97 | OTP | 0 | 0 | -24679 | 26598.3 | 65714.2 | 0 |
| 98 | LIVE | 0 | 0 | -14541.2 | -2479.3 | -2868.4 | 0 |
| 99 | WOP1 | 0 | 0 | -252614 | -110147.4 | 273830 | 0 |
| 100 | WOP2 | 0 | 0 | -252612 | -110142.1 | 420807.3 | 0 |
| 101 | WOP3 | 0 | 0 | -252614 | 220474.8 | 273832.5 | 0 |
| 102 | WOP4 | 0 | 0 | -252611 | 220435.9 | 420784.5 | 0 |
| 103 | WEX1 | 0 | 0 | -238107 | -127019.1 | 218557.3 | 0 |
| 104 | WEX2 | 0 | 0 | -238107 | -136563.6 | 368705 | 0 |
| 105 | WEX3 | 0 | 0 | -238107 | 223845.2 | 218554.9 | 0 |
| 106 | WEX4 | 0 | 0 | -238107 | 223839.4 | 397635.2 | 0 |
| 107 | WLT1 | 0 | 0 | -206831 | -141101.9 | 193869.4 | 0 |
| 108 | OPNW | 0 | 0 | -262299 | -87544.5 | 300048.5 | 0 |
| 109 | OPNE | 0 | 0 | -262296 | -87539.2 | 447025.7 | 0 |
| 110 | OPSW | 0 | 0 | -262299 | 243077.7 | 300050.9 | 0 |
| 111 | OPSE | 0 | 0 | -262296 | 243038.8 | 447002.9 | 0 |
| 112 | EXNW | 0 | 0 | -247792 | -104416.2 | 244775.8 | 0 |
| 113 | EXNE | 0 | 0 | -247792 | -113960.7 | 394923.5 | 0 |
| 114 | EXSW | 0 | 0 | -247792 | 246448.1 | 244773.4 | 0 |
| 115 | EXSE | 0 | 0 | -247792 | 246442.3 | 423853.7 | 0 |
| 116 | LTNW | 0 | 0 | -216516 | -118499 | 220087.9 | 0 |
| 117 | OH11 | 22913.06 | 8.19 | -318560 | -29538.6 | 3448220 | 16776.7 |
| 118 | OH12 | 16536.74 | 27702.46 | -319607 | -2962362.4 | 2897786 | 197954 |
| 119 | OH13 | -26.6 | 35235.08 | -319338 | -3792942.9 | 1285735 | 189130.2 |
| 120 | OH14 | -18331.1 | 30470.22 | -318607 | -3230745.3 | -490212 | 260966.3 |
| 121 | OH15 | -26457.8 | 5.42 | -317700 | -31286.5 | -1255568 | -13762.7 |
| 122 | OH16 | -18403.3 | -30670.4 | -318584 | 3187847.2 | -495145 | -276392 |
| 123 | OH17 | -34.63 | -35433.7 | -319382 | 3749522.2 | 1286936 | -190162 |
| 124 | OH18 | 16638.99 | -27819.4 | -319604 | 2912106.1 | 2905752 | -185781 |
| 125 | OL11 | 23505.74 | 11.88 | -320675 | -29951.1 | 3467486 | 18425.2 |
| 126 | OL12 | 17057.91 | 28534.96 | -321761 | -3005113.7 | 2929155 | 206259.9 |
| 127 | OL13 | -4.09 | 35964.24 | -321417 | -3804491 | 1292323 | 194521 |
| 128 | OL14 | -18792.5 | 31117.87 | -320620 | -3242354.4 | -504301 | 267253.9 |
| 129 | OL15 | -26791.6 | 12.33 | -319571 | -32247.6 | -1229952 | -14961.1 |
| 130 | OL16 | -18850 | -31327.9 | -320569 | 3199899.2 | -507359 | -284111 |
| 131 | OL17 | -12.91 | -36185.3 | -321434 | 3763073.9 | 1293589 | -195326 |
| 132 | OL18 | 17171.04 | -28662.7 | -321743 | 2955762.6 | 2938163 | -192622 |
| 133 | EH11 | 86739.56 | 35.74 | -305774 | -46973.4 | 9020392 | 72634.8 |
| 134 | EH12 | 57716.42 | 92862.11 | -305648 | -9174201.4 | 6484134 | 723952.3 |
| 135 | EH13 | 24.1 | 111458.3 | -304296 | -11056730 | 1233032 | 660877.3 |
| 136 | EH14 | -60414.4 | 96786.22 | -302577 | -9513262.1 | -4249023 | 754941.2 |
| 137 | EH15 | -90848.5 | 46.74 | -303726 | -52234.3 | -6922043 | -71416.4 |
| 138 | EH16 | -59734.1 | -95975.3 | -302680 | 9340631.6 | -4187418 | -816624 |
| 139 | EH17 | -27.69 | -111293 | -304556 | 10943537.7 | 1234733 | -659200 |
| 140 | EH18 | 2848.23 | -105091 | -304493 | 10250027.3 | 1672113 | -631124 |
| 141 | EL11 | 90287.71 | 27.96 | -309033 | -45715.4 | 9220545 | 79117.8 |
| 142 | EL12 | 60316.53 | 96911.19 | -308421 | -9451711.6 | 6645629 | 751613.8 |
| 143 | EL13 | 23.94 | 116442.5 | -306910 | -11402181 | 1237290 | 686178.5 |
| 144 | EL14 | -62976.9 | 100790.1 | -307414 | -9799581.9 | -4408534 | 740553.5 |
| 145 | EL15 | -94293.7 | 39.31 | -305510 | -52405.9 | -7079511 | -75876.3 |
| 146 | EL16 | -62328.1 | -99926.3 | -307501 | 9624032.3 | -4347503 | -808961 |
| 147 | EL17 | -26.09 | -116268 | -307188 | 11287602.9 | 1239931 | -684471 |
| 148 | EL18 | 2849.75 | -109851 | -307130 | 10579210.5 | 1677297 | -655302 |
| 149 | OH21 | 22913.06 | 8.19 | -318558 | -29533.4 | 3595197 | 16776.7 |
| 150 | OH22 | 16536.74 | 27702.46 | -319604 | -2962357.2 | 3044764 | 197954 |
| 151 | OH23 | -26.6 | 35235.08 | -319335 | -3792937.6 | 1432712 | 189130.2 |
| 152 | OH24 | -18331.1 | 30470.22 | -318604 | -3230740 | -343234 | 260966.3 |
| 153 | OH25 | -26457.8 | 5.42 | -317697 | -31281.3 | -1108590 | -13762.7 |
| 154 | OH26 | -18403.3 | -30670.4 | -318581 | 3187852.5 | -348167 | -276392 |
| 155 | OH27 | -34.63 | -35433.7 | -319380 | 3749527.5 | 1433913 | -190162 |
| 156 | OH28 | 16638.99 | -27819.4 | -319602 | 2912111.4 | 3052729 | -185781 |
| 157 | OL21 | 23505.74 | 11.88 | -320672 | -29945.9 | 3614463 | 18425.2 |
| 158 | OL22 | 17057.91 | 28534.96 | -321758 | -3005108.5 | 3076133 | 206259.9 |
| 159 | OL23 | -4.09 | 35964.24 | -321414 | -3804485.7 | 1439301 | 194521 |
| 160 | OL24 | -18792.5 | 31117.87 | -320617 | -3242349.1 | -357323 | 267253.9 |
| 161 | OL25 | -26791.6 | 12.33 | -319569 | -32242.4 | -1082975 | -14961.1 |
| 162 | OL26 | -18850 | -31327.9 | -320567 | 3199904.4 | -360382 | -284111 |
| 163 | OL27 | -12.91 | -36185.3 | -321431 | 3763079.2 | 1440566 | -195326 |
| 164 | OL28 | 17171.04 | -28662.7 | -321741 | 2955767.8 | 3085141 | -192622 |
| 165 | EH21 | 86739.56 | 35.74 | -305773 | -56517.9 | 9170540 | 72634.8 |
| 166 | EH22 | 57716.42 | 92862.11 | -305647 | -9183745.8 | 6634281 | 723952.3 |
| 167 | EH23 | 24.1 | 111458.3 | -304295 | -11066274 | 1383180 | 660877.3 |
| 168 | EH24 | -60414.4 | 96786.22 | -302576 | -9522806.6 | -4098875 | 754941.2 |
| 169 | EH25 | -90848.5 | 46.74 | -303725 | -61778.8 | -6771895 | -71416.4 |
| 170 | EH26 | -59734.1 | -95975.3 | -302679 | 9331087.1 | -4037270 | -816624 |
| 171 | EH27 | -27.69 | -111293 | -304556 | 10933993.2 | 1384881 | -659200 |
| 172 | EH28 | 57992.76 | -93191.6 | -305775 | 9100193.3 | 6659988 | -657509 |
| 173 | EL21 | 90287.71 | 27.96 | -309032 | -55259.9 | 9370692 | 79117.8 |
| 174 | EL22 | 60316.53 | 96911.19 | -308421 | -9461256.1 | 6795776 | 751613.8 |
| 175 | EL23 | 23.94 | 116442.5 | -306909 | -11411726 | 1387438 | 686178.5 |
| 176 | EL24 | -62976.9 | 100790.1 | -307413 | -9809126.4 | -4258387 | 740553.5 |
| 177 | EL25 | -94293.7 | 39.31 | -305509 | -61950.4 | -6929363 | -75876.3 |
| 178 | EL26 | -62328.1 | -99926.3 | -307500 | 9614487.8 | -4197355 | -808961 |
| 179 | EL27 | -26.09 | -116268 | -307187 | 11278058.4 | 1390079 | -684471 |
| 180 | EL28 | 60616.56 | -97266.8 | -308550 | 9379902.5 | 6823922 | -678902 |
| 181 | OH31 | 22913.06 | 8.19 | -318560 | 301083.5 | 3448223 | 16776.7 |
| 182 | OH32 | 16536.74 | 27702.46 | -319607 | -2631740.3 | 2897789 | 197954 |
| 183 | OH33 | -26.6 | 35235.08 | -319338 | -3462320.8 | 1285737 | 189130.2 |
| 184 | OH34 | -18331.1 | 30470.22 | -318607 | -2900123.1 | -490209 | 260966.3 |
| 185 | OH35 | -26457.8 | 5.42 | -317700 | 299335.6 | -1255565 | -13762.7 |
| 186 | OH36 | -18403.3 | -30670.4 | -318584 | 3518469.4 | -495142 | -276392 |
| 187 | OH37 | -34.63 | -35433.7 | -319382 | 4080144.4 | 1286939 | -190162 |
| 188 | OH38 | 16638.99 | -27819.4 | -319604 | 3242728.3 | 2905755 | -185781 |
| 189 | OL31 | 23505.74 | 11.88 | -320675 | 300671 | 3467488 | 18425.2 |
| 190 | OL32 | 17057.91 | 28534.96 | -321761 | -2674491.6 | 2929158 | 206259.9 |
| 191 | OL33 | -4.09 | 35964.24 | -321417 | -3473868.8 | 1292326 | 194521 |
| 192 | OL34 | -18792.5 | 31117.87 | -320620 | -2911732.2 | -504298 | 267253.9 |
| 193 | OL35 | -26791.6 | 12.33 | -319572 | 298374.5 | -1229950 | -14961.1 |
| 194 | OL36 | -18850 | -31327.9 | -320570 | 3530521.3 | -507357 | -284111 |
| 195 | OL37 | -12.91 | -36185.3 | -321434 | 4093696.1 | 1293592 | -195326 |
| 196 | OL38 | 17171.04 | -28662.7 | -321744 | 3286384.7 | 2938166 | -192622 |
| 197 | EH31 | 86739.56 | 35.74 | -305774 | 303890.8 | 9020390 | 72634.8 |
| 198 | EH32 | 57716.42 | 92862.11 | -305648 | -8823337.1 | 6484131 | 723952.3 |
| 199 | EH33 | 24.1 | 111458.3 | -304296 | -10705865 | 1233029 | 660877.3 |
| 200 | EH34 | -60414.4 | 96786.22 | -302577 | -9162397.9 | -4249025 | 754941.2 |
| 201 | EH35 | -90848.5 | 46.74 | -303726 | 298629.9 | -6922045 | -71416.4 |
| 202 | EH36 | -59734.1 | -95975.3 | -302680 | 9691495.8 | -4187420 | -816624 |
| 203 | EH37 | -27.69 | -111293 | -304556 | 11294401.9 | 1234731 | -659200 |
| 204 | EH38 | 57992.76 | -93191.6 | -305776 | 9460602 | 6509838 | -657509 |
| 205 | EL31 | 90287.71 | 27.96 | -309033 | 305148.9 | 9220542 | 79117.8 |
| 206 | EL32 | 60316.53 | 96911.19 | -308421 | -9100847.4 | 6645626 | 751613.8 |
| 207 | EL33 | 23.94 | 116442.5 | -306910 | -11051317 | 1237288 | 686178.5 |
| 208 | EL34 | -62976.9 | 100790.1 | -307414 | -9448717.6 | -4408537 | 740553.5 |
| 209 | EL35 | -94293.7 | 39.31 | -305510 | 298458.3 | -7079513 | -75876.3 |
| 210 | EL36 | -62328.1 | -99926.3 | -307501 | 9974896.5 | -4347506 | -808961 |
| 211 | EL37 | -26.09 | -116268 | -307188 | 11638467.1 | 1239929 | -684471 |
| 212 | EL38 | 60616.56 | -97266.8 | -308551 | 9740311.3 | 6673772 | -678902 |
| 213 | OH41 | 22913.06 | 8.19 | -318557 | 301044.7 | 3595175 | 16776.7 |
| 214 | OH42 | 16536.74 | 27702.46 | -319604 | -2631779.1 | 3044741 | 197954 |
| 215 | OH43 | -26.6 | 35235.08 | -319334 | -3462359.6 | 1432689 | 189130.2 |
| 216 | OH44 | -18331.1 | 30470.22 | -318604 | -2900162 | -343257 | 260966.3 |
| 217 | OH45 | -26457.8 | 5.42 | -317697 | 299296.8 | -1108613 | -13762.7 |
| 218 | OH46 | -18403.3 | -30670.4 | -318581 | 3518430.5 | -348190 | -276392 |
| 219 | OH47 | -34.63 | -35433.7 | -319379 | 4080105.5 | 1433891 | -190162 |
| 220 | OH48 | 16638.99 | -27819.4 | -319601 | 3242689.4 | 3052707 | -185781 |
| 221 | OL41 | 23505.74 | 11.88 | -320671 | 300632.2 | 3614440 | 18425.2 |
| 222 | OL42 | 17057.91 | 28534.96 | -321758 | -2674530.4 | 3076110 | 206259.9 |
| 223 | OL43 | -4.09 | 35964.24 | -321413 | -3473907.7 | 1439278 | 194521 |
| 224 | OL44 | -18792.5 | 31117.87 | -320616 | -2911771.1 | -357346 | 267253.9 |
| 225 | OL45 | -26791.6 | 12.33 | -319568 | 298335.7 | -1082998 | -14961.1 |
| 226 | OL46 | -18850 | -31327.9 | -320566 | 3530482.5 | -360405 | -284111 |
| 227 | OL47 | -12.91 | -36185.3 | -321430 | 4093657.2 | 1440544 | -195326 |
| 228 | OL48 | 17171.04 | -28662.7 | -321740 | 3286345.9 | 3085118 | -192622 |
| 229 | EH41 | 86739.56 | 35.74 | -305773 | 303885.1 | 9199470 | 72634.8 |
| 230 | EH42 | 57716.42 | 92862.11 | -305647 | -8823342.9 | 6663212 | 723952.3 |
| 231 | EH43 | 24.1 | 111458.3 | -304295 | -10705871 | 1412110 | 660877.3 |
| 232 | EH44 | -60414.4 | 96786.22 | -302576 | -9162403.6 | -4069945 | 754941.2 |
| 233 | EH45 | -90848.5 | 46.74 | -303725 | 298624.2 | -6742965 | -71416.4 |
| 234 | EH46 | -59734.1 | -95975.3 | -302679 | 9691490 | -4008340 | -816624 |
| 235 | EH47 | -27.69 | -111293 | -304556 | 11294396.1 | 1413811 | -659200 |
| 236 | EH48 | 57992.76 | -93191.6 | -305775 | 9460596.3 | 6688918 | -657509 |
| 237 | EL41 | 90287.71 | 27.96 | -309033 | 305143.1 | 9399622 | 79117.8 |
| 238 | EL42 | 60316.53 | 96911.19 | -308421 | -9100853.1 | 6824707 | 751613.8 |
| 239 | EL43 | 23.94 | 116442.5 | -306910 | -11051323 | 1416368 | 686178.5 |
| 240 | EL44 | -62976.9 | 100790.1 | -307413 | -9448723.4 | -4229456 | 740553.5 |
| 241 | EL45 | -94293.7 | 39.31 | -305509 | 298452.6 | -6900433 | -75876.3 |
| 242 | EL46 | -62328.1 | -99926.3 | -307501 | 9974890.8 | -4168425 | -808961 |
| 243 | EL47 | -26.09 | -116268 | -307187 | 11638461.4 | 1419009 | -684471 |
| 244 | EL48 | 60616.56 | -97266.8 | -308551 | 9740305.5 | 6852852 | -678902 |
| 245 | LH11 | 86739.56 | 35.74 | -274498 | -61056.3 | 8995704 | 72634.8 |
| 246 | LH12 | 57716.42 | 92862.11 | -274372 | -9188284.2 | 6459446 | 723952.3 |
| 247 | LH13 | 24.1 | 111458.3 | -273020 | -11070812 | 1208344 | 660877.3 |
| 248 | LH14 | -60414.4 | 96786.22 | -271301 | -9527345 | -4273711 | 754941.2 |
| 249 | LH15 | -90848.5 | 46.74 | -272450 | -66317.1 | -6946731 | -71416.4 |
| 250 | LH16 | -59734.1 | -95975.3 | -271404 | 9326548.7 | -4212106 | -816624 |
| 251 | LH17 | -27.69 | -111293 | -273280 | 10929454.8 | 1210045 | -659200 |
| 252 | LH18 | 57992.76 | -93191.6 | -274500 | 9095655 | 6485152 | -657509 |
| 253 | LL11 | 90287.71 | 27.96 | -277757 | -59798.2 | 9195857 | 79117.8 |
| 254 | LL12 | 60316.53 | 96911.19 | -277145 | -9465794.5 | 6620941 | 751613.8 |
| 255 | LL13 | 23.94 | 116442.5 | -275634 | -11416264 | 1212602 | 686178.5 |
| 256 | LL14 | -62976.9 | 100790.1 | -276138 | -9813664.7 | -4433222 | 740553.5 |
| 257 | LL15 | -94293.7 | 39.31 | -274234 | -66488.8 | -7104199 | -75876.3 |
| 258 | LL16 | -62328.1 | -99926.3 | -276225 | 9609949.4 | -4372191 | -808961 |
| 259 | LL17 | -26.09 | -116268 | -275912 | 11273520 | 1215243 | -684471 |
| 260 | LL18 | 60616.56 | -97266.8 | -277275 | 9375364.2 | 6649086 | -678902 |

分析结果

分析结果汇总信息如下表所示。计算结果显示构件、节点冲剪和桩应力值均小于1.0，操作工况桩基安全系数大于2.0，极端工况桩基安全系数大于1.5，结构强度满足规范要求，本次结构强度评估结论通过。（内置参考结论性语句）

分析汇总信息（来自结果读取结果的汇总信息）

| 校核内容 | 位置 | 最大UC值/ 最小安全系数 | 对应工况 | 是否满足 |
| --- | --- | --- | --- | --- |
| 构件 |  |  |  |  |
| 节点冲剪 |  |  |  |  |
| 桩应力 |  |  |  |  |
| 操作工况桩基抗压 |  |  |  |  |
| 操作工况桩基抗拔 |  |  |  |  |
| 极端工况桩基抗压 |  |  |  |  |
| 极端工况桩基抗拔 |  |  |  |  |

构件名义应力校核

根据API RP 2A-WSD（校核规范名称通过结果文件读取可获得）进行名义应力校核。对极端风暴条件，容许应力被增加三分之一。名义应力校核结果显示，下列导管架部分杆件UC值均小于1.0。所有杆件应力比结果如下：

（以下内容来自结果文件读取）

* * *  M E M B E R  G R O U P  S U M M A R Y  * * *

API RP2A 21ST/AISC 9TH

MAX.  DIST                                                                  EFFECTIVE      CM

GRUP CRITICAL LOAD  UNITY  FROM   * APPLIED STRESSES *    *** ALLOWABLE STRESSES ***   CRIT       LENGTHS    * VALUES *

ID   MEMBER  COND  CHECK  END     AXIAL BEND-Y BEND-Z    AXIAL  EULER BEND-Y BEND-Z   COND     KLY    KLZ    Y     Z

M       N/MM2  N/MM2  N/MM2    N/MM2  N/MM2  N/MM2  N/MM2            M      M

CA   CA9-191L EXL2   0.01   1.9     0.00  -0.54   1.52  .19E+03.57E+04.23E+03.23E+03   TN+BN     1.9    1.9  0.85  0.85

CN2 139C-239C EXL1   0.24   0.0   -18.31   0.05  56.34  .22E+03.40E+03.35E+03.35E+03   C<.15    10.0   10.0  0.85  0.85

CN3 239C-339C OPL3   0.43   0.0   -18.75  -6.33   0.11  .47E+02.47E+02.27E+03.27E+03   C>.15A   25.2   25.2  0.85  0.85

CN4 339C-439C EXH1   0.41  25.0   -13.92  -0.39  61.73  .64E+02.64E+02.35E+03.35E+03   C>.15A   25.0   25.0  0.85  0.85

CN5 439C-539C EXL1   0.33   0.0   -13.07  -0.37  61.77  .83E+02.83E+02.35E+03.35E+03   C>.15A   22.0   22.0  0.85  0.85

CN6 539C-639C EXL1   0.31  20.0    -9.42  -3.21  78.11  .10E+03.10E+03.35E+03.35E+03   C<.15    20.0   20.0  0.85  0.85

CN7 639C-739C EXL1   0.26   0.0    -9.41  -3.03  69.32  .14E+03.14E+03.35E+03.35E+03   C<.15    17.0   17.0  0.85  0.85

CN8 739C-839C OPL2   0.20  11.0    -8.37 -38.62  -0.12  .15E+03.25E+03.27E+03.27E+03   C<.15    11.0   11.0  0.85  0.85

CN9 326C-426C EXL1   0.22  25.0    -6.16  -1.46  63.90  .18E+03.22E+03.34E+03.34E+03   C<.15    25.0   25.0  0.85  0.85

H70 L20H-L10H OPH4   0.19   3.2    -3.05  31.60   6.02  .17E+03.45E+03.21E+03.26E+03   C<.15     3.2    3.2  0.85  0.85

J10 191L-J102 EXL1   0.54   9.5    21.37 149.88 -23.11  .28E+03.13E+03.33E+03.33E+03   TN+BN    27.3   11.5  0.85  0.85

J11 119L-J105 EXL9   0.51   0.0   -43.65 -41.64 -56.95  .15E+03.15E+03.33E+03.33E+03   C>.15A   25.3   11.5  0.85  0.85

J12 J104-J106 EXH7   0.32   0.9    56.93  21.61  31.50  .28E+03.15E+03.33E+03.33E+03   TN+BN    25.3    9.1  0.85  0.85

J13 J106-J102 EXL1   0.26  16.6     6.52  64.66 -45.42  .28E+03.35E+03.33E+03.33E+03   TN+BN    13.2   13.2  0.85  0.85

J14 J102-J103 EXL1   0.82   0.5    26.70 240.99 -51.22  .28E+03.13E+03.34E+03.34E+03   TN+BN    27.3   11.1  0.85  0.85

J20 211L-J200 EXL1   0.39   0.0   -38.47  -2.66  62.17  .18E+03.22E+03.33E+03.33E+03   C>.15A   25.3   16.1  0.85  0.85

J21 211L-J203 EXL1   0.24   0.0     9.12  54.62  42.42  .28E+03.43E+03.33E+03.33E+03   TN+BN    17.9   17.9  0.85  0.85

J22 J200-J204 EXL8   0.17   0.0    -7.63   8.61 -41.35  .20E+03.30E+03.33E+03.33E+03   C<.15    17.7   17.7  0.85  0.85

J23 J203-219L EXL7   0.22  15.0     8.98  29.47  55.75  .28E+03.66E+03.34E+03.34E+03   TN+BN    12.0   12.0  0.85  0.85

J24 219L-J201 EXL7   0.41   0.0   -12.34 -19.67-119.78  .18E+03.21E+03.35E+03.35E+03   C<.15    25.3    8.1  0.85  0.85

J25 J203-J205 EXL5   0.09   7.7    -3.27  14.53  23.70  .24E+03.78E+03.34E+03.34E+03   C<.15    13.4    8.5  0.85  0.85

J26 J205-J201 EXL7   0.15   4.2    -5.08  14.34 -43.38  .24E+03.77E+03.34E+03.34E+03   C<.15    13.4    5.0  0.85  0.85

J28 J205-J216 EXL4   0.17   0.0     1.64   5.00  53.65  .28E+03.61E+03.34E+03.34E+03   TN+BN    15.0    5.5  0.85  0.85

J29 J204-J220 EXL9   0.27   3.0    -0.81   8.70 -31.49  .31E+03.52E+03.35E+03.35E+03   L.BEND   15.0    3.5  0.85  0.85

J2A J209-J220 EXL3   0.17   2.3    -5.23  -8.50 -48.41  .19E+03.27E+03.35E+03.35E+03   C<.15    15.0    2.3  0.85  0.85

J2B J210-J203 OPL6   0.00   1.1     0.00  -0.07   0.00  .17E+03.45E+03.27E+03.27E+03   SHEAR    17.2    1.1  0.85  0.85

J2C J201-J212 EXL4   0.00   0.0     0.00  -0.05   0.05  .23E+03.59E+03.35E+03.35E+03   SHEAR    15.0    1.0  0.85  0.85

J2D J216-J215 EXL5   0.05   0.0    -1.96   3.91 -12.26  .23E+03.60E+03.35E+03.35E+03   C<.15    15.0    1.5  0.85  0.85

J2E J215-J214 EXL9   0.06   2.9     1.69  12.63  14.20  .28E+03.62E+03.34E+03.34E+03   TN+BN    15.0    3.6  0.85  0.85

J2F J217-J206 EXL6   0.08   0.7    -4.74  19.28  -3.35  .26E+03.19E+04.34E+03.34E+03   C<.15     8.5    4.3  0.85  0.85

J2G J204-J217 EXL4   0.10   4.2    -0.58   7.17  32.59  .26E+03.20E+04.35E+03.35E+03   C<.15     8.5    4.2  0.85  0.85

J30 391L-J300 EXL7   0.42   0.0    51.11  23.95  74.99  .28E+03.13E+03.33E+03.33E+03   TN+BN    27.1   13.6  0.85  0.85

J31 391L-J303 EXL2   0.47   0.0    45.89  20.27 -98.71  .28E+03.11E+03.33E+03.33E+03   TN+BN    29.1   14.6  0.85  0.85

J32 J300-J302 EXL2   0.25   0.0   -17.89   3.95  51.52  .19E+03.24E+03.33E+03.33E+03   C<.15    16.0   16.0  0.85  0.85

J33 J303-J305 EXL7   0.54   0.0    -9.60  -2.82-169.82  .18E+03.24E+03.35E+03.35E+03   C<.15    16.0    4.8  0.85  0.85

J34 J305-J301 EXL7   0.50   1.0   -14.23   6.42 139.05  .19E+03.24E+03.33E+03.33E+03   C<.15    16.0   15.3  0.85  0.85

J35 J304-J309 EXL7   0.63   0.0   -10.68  -4.02 176.52  .92E+02.92E+02.34E+03.34E+03   C<.15    25.6   25.6  0.85  0.85

J40 411L-J400 EXL5   0.39   0.0    62.77  32.35 -46.10  .28E+03.20E+03.33E+03.33E+03   TN+BN    22.1   11.1  0.85  0.85

J41 J402-419L EXL7   0.37   0.0    45.54  -0.28 -70.67  .28E+03.22E+04.34E+03.34E+03   TN+BN     6.6    6.6  0.85  0.85

J42 J400-J402 EXL2   0.29   0.0   -28.16  -1.33  52.72  .22E+03.49E+03.33E+03.33E+03   C<.15    13.9   13.9  0.85  0.85

J43 J402-J404 EXL6   0.21   0.0    -0.02   9.24 -73.92  .26E+03.15E+04.35E+03.35E+03   C<.15     8.5    8.5  0.85  0.85

J44 J402-J401 EXL3   0.38  10.2   -76.35  -3.74  20.07  .23E+03.63E+03.34E+03.34E+03   C>.15A    8.1    8.1  0.85  0.85

J45 J405-499L EXLA   0.25   3.1    29.52  30.52  42.25  .28E+03.27E+03.35E+03.35E+03   TN+BN    22.1    3.1  0.85  0.85

J46 J404-J401 EXL2   0.27   0.0    48.52  30.08  13.23  .28E+03.28E+03.34E+03.34E+03   TN+BN    22.1    7.0  0.85  0.85

J47 J402-J413 EXL6   0.68   3.0    14.37   0.12  68.65  .28E+03.23E+03.35E+03.35E+03   L.BEND   15.0   15.0  0.85  0.85

J48 J406-J407 EXL6   0.47   0.0    27.59   2.76-128.62  .28E+03.27E+03.35E+03.35E+03   TN+BN    15.0   15.0  0.85  0.85

J50 591L-J500 EXH7   0.50   0.0    89.42  14.07  59.29  .28E+03.19E+03.33E+03.33E+03   TN+BN    18.0   18.0  0.85  0.85

J51 511L-J508 EXHA   0.33   0.0    54.01  15.04  47.17  .28E+03.13E+03.35E+03.35E+03   TN+BN    21.7   12.0  0.85  0.85

J52 J509-J507 EXH2   0.25   3.7    61.02  -5.78 -11.50  .28E+03.13E+03.35E+03.35E+03   TN+BN    21.7    4.9  0.85  0.85

J53 J507-599L EXL5   0.56   3.2    76.54  28.15 -94.03  .28E+03.15E+03.34E+03.34E+03   TN+BN    21.7    3.2  0.85  0.85

J54 J503-599L EXLA   0.41   1.1    41.49  15.00  91.58  .28E+03.15E+03.35E+03.35E+03   TN+BN    19.7    1.1  0.85  0.85

J55 J502-J501 EXL1   0.30   5.7    63.20  -3.52  26.74  .28E+03.30E+03.35E+03.35E+03   TN+BN    14.0    7.0  0.85  0.85

J56 J519-J512 EXL1   0.50   2.3   -19.62   3.34 136.93  .19E+03.27E+03.34E+03.34E+03   C<.15    15.0    2.3  0.85  0.85

J57 J504-J520 EXL6   0.64   4.0    -9.97   6.76-195.83  .25E+03.13E+04.33E+03.33E+03   C<.15     6.9    6.9  0.85  0.85

J58 J510-J504 EXL7   0.11   4.3     3.82   3.28  31.98  .28E+03.10E+04.33E+03.33E+03   TN+BN    12.0    5.3  0.85  0.85

J59 J504-J502 EXLA   0.15   1.6   -10.20  -8.12  36.07  .25E+03.10E+04.33E+03.33E+03   C<.15    12.0    6.8  0.85  0.85

J5A J514-J505 EXL6   0.66   3.0    16.75  -0.14 213.56  .28E+03.26E+03.35E+03.35E+03   TN+BN    15.0    3.0  0.85  0.85

J5B J515-J510 EXL9   0.00   1.5     0.00   0.08  -0.75  .23E+03.64E+03.34E+03.34E+03   SHEAR    14.6    1.5  0.85  0.85

J60 691L-J600 EXL9   0.58   0.6    82.05  -2.03  96.32  .28E+03.32E+03.33E+03.33E+03   TN+BN    14.0    7.0  0.85  0.85

J61 699L-J602 EXLA   0.44   0.6    69.41  15.28 -61.22  .28E+03.34E+03.33E+03.33E+03   TN+BN    14.0    4.2  0.85  0.85

J62 J601-J602 EXL2   0.55   0.6    68.66   0.72 101.65  .28E+03.32E+03.33E+03.33E+03   TN+BN    14.0    4.6  0.85  0.85

J63 691L-J604 OPL1   0.41   0.0    67.38  -6.41 -23.12  .21E+03.18E+03.25E+03.25E+03   TN+BN    16.0    9.7  0.85  0.85

J64 J604-699L EXL5   0.52   5.7    87.16  14.37 -67.20  .28E+03.25E+03.33E+03.33E+03   TN+BN    16.0    6.4  0.85  0.85

J65 J617-J601 EXL5   0.66   3.3   -66.27  -8.65 143.84  .28E+03.10E+04.34E+03.34E+03   C>.15B    6.4    6.4  0.85  0.85

J66 J616-J605 EXL1   0.43   0.0   -33.40  21.95  91.49  .18E+03.23E+03.34E+03.34E+03   C>.15A   15.0    6.1  0.85  0.85

J67 J605-J607 EXL1   0.23   0.0   -19.01  10.63 -38.68  .16E+03.18E+03.35E+03.35E+03   C<.15    15.0    4.6  0.85  0.85

J68 J607-J608 EXL1   0.29   0.5   -33.78  21.57  25.81  .17E+03.20E+03.34E+03.34E+03   C>.15A   15.0    4.6  0.85  0.85

J69 J605-J610 EXL1   0.17   0.0   -22.62   2.72 -28.09  .25E+03.93E+03.35E+03.35E+03   C<.15     6.6    1.0  0.85  0.85

J6A J613-J602 EXL7   0.80   3.3    36.26  -4.01-227.18  .28E+03.97E+03.34E+03.34E+03   TN+BN     6.6    3.3  0.85  0.85

J6B J600-J954 EXL2   0.70   0.0   -38.69 -17.83 176.69  .20E+03.30E+03.34E+03.34E+03   C>.15A   11.9   11.9  0.85  0.85

J70 791L-J700 OPH9   0.28   4.5    44.92  15.32  -7.06  .21E+03.44E+03.24E+03.24E+03   TN+BN    10.9    5.5  0.85  0.85

J71 791L-J707 OPH1   0.24   0.0    16.98 -35.72 -15.31  .21E+03.27E+03.24E+03.24E+03   TN+BN    12.9    7.8  0.85  0.85

J72 J706-719L EXH6   0.33   5.1     4.55 -24.63 101.55  .28E+03.31E+03.34E+03.34E+03   TN+BN    12.9    5.1  0.85  0.85

J73 719L-J701 EXH5   0.72   0.0    17.75 -46.34 206.29  .28E+03.53E+03.32E+03.32E+03   TN+BN    10.9    3.7  0.85  0.85

J74 J705-799L EXH7   0.43   1.5    24.91 -28.33 116.21  .28E+03.49E+03.35E+03.35E+03   TN+BN    10.9    2.6  0.85  0.85

J75 J706-J717 EXH1   0.71   0.0   -12.71 -30.87-197.81  .20E+03.31E+03.31E+03.31E+03   C<.15    11.6    4.1  0.85  0.85

J76 J707-J709 EXH6   0.54   0.0    -7.83  -0.53-156.45  .20E+03.31E+03.31E+03.31E+03   C<.15    11.6    3.0  0.85  0.85

J77 J708-J709 EXHA   0.38   0.6    -6.80  31.86 102.15  .20E+03.31E+03.31E+03.31E+03   C<.15    11.6    4.6  0.85  0.85

J78 J701-J711 EXHA   0.77   0.0   -43.33  -5.34 192.25  .28E+03.16E+04.31E+03.31E+03   C>.15B    5.2    1.6  0.85  0.85

J79 J718-J710 EXHA   0.34   0.3     7.34  12.54 108.57  .28E+03.15E+04.35E+03.35E+03   TN+BN     5.2    0.3  0.85  0.85

K10 J107-299L EXL9   0.78   0.0  -154.03  18.97  16.75  .22E+03.50E+03.33E+03.33E+03   C>.15A   11.1   11.1  0.85  0.85

K20 K11L-J900 EXL1   0.18   0.0   -22.15  -5.38  29.59  .24E+03.66E+03.34E+03.34E+03   C<.15    14.4   14.4  0.85  0.85

K21 K11L-J902 EXL3   0.22   0.0   -12.50  20.91 -50.62  .23E+03.51E+03.34E+03.34E+03   C<.15    13.6   13.6  0.85  0.85

K30 J920-J405 EXLA   0.32   3.4   -11.63  35.36 -89.75  .26E+03.23E+04.35E+03.35E+03   C<.15     3.4    3.4  0.85  0.85

K40 J405-J923 EXL1   0.28   0.0   -18.33  73.38 -13.15  .26E+03.21E+04.35E+03.35E+03   C<.15     3.6    3.6  0.85  0.85

LG2 111L-211L EXL7   0.66   7.9   -99.69  34.31  87.97  .28E+03.67E+04.31E+03.31E+03   C>.15B   10.0   10.0  0.85  0.85

LG3 211L-K11L EXL7   0.95   2.4  -188.14 -48.03 -40.04  .25E+03.10E+04.32E+03.32E+03   C>.15A   25.4   13.7  0.85  0.85

LG4 K11L-311L EXL7   0.87   1.1  -187.66  34.15   9.21  .25E+03.10E+04.32E+03.32E+03   C>.15A   25.4   11.7  0.85  0.85

LG5 391L-491L EXL5   0.81  23.1  -187.08  11.11   6.37  .24E+03.10E+04.31E+03.31E+03   C>.15A   25.2   25.2  0.85  0.85

LG6 411L-511L EXH7   0.87  20.3  -194.87  19.81   1.38  .24E+03.97E+03.31E+03.31E+03   C>.15A   22.2   22.2  0.85  0.85

LG7 591L-691L OPH5   0.72   2.3  -128.26   3.43  -1.46  .18E+03.88E+03.23E+03.23E+03   C>.15A   20.1   20.1  0.85  0.85

LG9 791L-891L OPL4   0.42   1.5   -65.38  30.96  -2.43  .20E+03.34E+04.27E+03.27E+03   C>.15A    8.0    8.0  0.85  0.85

LGA 624L-614L OPL9   0.64   3.3  -112.03  -3.15   7.97  .19E+03.79E+03.24E+03.24E+03   C>.15A   17.1   17.1  0.85  0.85

LGB 620L-791L OPH4   0.68   4.5  -122.98  -1.67  -4.67  .19E+03.80E+03.24E+03.24E+03   C>.15A   17.1   17.1  0.85  0.85

LGC 691L-620L OPH4   0.62   6.5  -117.56  -1.45  -2.53  .19E+03.12E+04.24E+03.24E+03   C>.15A   17.1   17.1  0.85  0.85

LGD 891L-LL21 OPL4   0.42   0.0   -65.43  20.36 -23.43  .21E+03.39E+04.27E+03.27E+03   C>.15B    8.0    8.0  0.85  0.85

LGG 299L-399L EXL2   0.92   2.4  -188.67 -31.63 -34.45  .25E+03.10E+04.32E+03.32E+03   C>.15A   25.4   25.5  0.85  0.85

SK2 131S-231S EXL3   0.34   0.0   -16.96 -66.91 -50.40  .26E+03.81E+04.30E+03.30E+03   C<.15    10.0   10.0  0.85  0.85

X20 291L-J901 EXL6   0.57   0.0   -90.45 -42.23  34.91  .23E+03.51E+03.33E+03.33E+03   C>.15A   19.2   19.2  0.85  0.85

X21 J901-399L EXL6   0.42  18.1   -77.69  -4.16 -25.65  .23E+03.51E+03.33E+03.33E+03   C>.15A   19.2   19.2  0.85  0.85

X22 J901-299L EXL1   0.60  20.8   -87.78 -54.28  39.17  .23E+03.51E+03.33E+03.33E+03   C>.15A   19.2   19.2  0.85  0.85

X23 211L-J902 EXL8   0.67   0.0  -111.79 -11.41  41.02  .22E+03.40E+03.33E+03.33E+03   C>.15A   18.5   18.5  0.85  0.85

X24 J902-291L EXL3   0.65  20.0  -105.88 -15.63  40.79  .22E+03.40E+03.33E+03.33E+03   C>.15A   18.5   18.5  0.85  0.85

X25 219L-J903 EXL9   0.65   0.0  -111.06 -11.06 -36.79  .22E+03.40E+03.33E+03.33E+03   C>.15A   18.5   18.5  0.85  0.85

X26 J903-299L EXL4   0.63  20.0  -107.83 -10.85 -36.30  .22E+03.40E+03.33E+03.33E+03   C>.15A   18.5   18.5  0.85  0.85

X30 399L-J907 EXL4   0.59   0.0  -115.32  -2.90  24.49  .23E+03.51E+03.33E+03.33E+03   C>.15A   16.5   16.5  0.85  0.85

X31 319L-J907 EXL9   0.61   0.0  -118.43  -3.06 -24.64  .23E+03.50E+03.33E+03.33E+03   C>.15A   16.5   16.5  0.85  0.85

X32 J921-J907 EXL3   0.57   0.0  -113.49  18.37  11.29  .23E+03.51E+03.33E+03.33E+03   C>.15A   16.5   16.5  0.85  0.85

X40 591L-J910 EXL9   0.60   0.0  -106.06  23.92  42.96  .24E+03.71E+03.33E+03.33E+03   C>.15A   13.9   13.9  0.85  0.85

X41 511L-J910 EXL4   0.57   0.0  -102.06  20.89 -40.84  .24E+03.71E+03.33E+03.33E+03   C>.15A   13.9   13.9  0.85  0.85

X50 611L-J914 EXL4   0.68   0.0  -123.72  30.43 -38.57  .24E+03.66E+03.33E+03.33E+03   C>.15A   12.1   12.1  0.85  0.85

X51 J914-691L EXL9   0.69  10.3  -129.16  18.63 -38.34  .23E+03.65E+03.33E+03.33E+03   C>.15A   12.1   12.1  0.85  0.85

X60 J931-J917 EXH2   0.38   3.4   -70.89   9.84  29.57  .24E+03.86E+03.34E+03.34E+03   C>.15A   10.4   10.4  0.85  0.85

X61 711L-J918 EXH4   0.37   0.0   -77.77  -5.20 -17.52  .25E+03.96E+03.34E+03.34E+03   C>.15A    9.8    9.8  0.85  0.85

X62 J917-791L EXH2   0.42   9.4   -68.54  43.37 -28.25  .24E+03.85E+03.34E+03.34E+03   C>.15A   10.4   10.4  0.85  0.85

X63 J925-J918 EXH4   0.42   3.7   -82.15  26.99  14.17  .25E+03.96E+03.34E+03.34E+03   C>.15A    9.8    9.8  0.85  0.85

X64 611L-J928 EXH7   0.30   0.0   -45.40  -7.49 -44.97  .24E+03.87E+03.35E+03.35E+03   C>.15A   10.4   10.4  0.85  0.85

X65 611L-J924 EXH9   0.38   0.0   -75.11 -11.31  25.52  .25E+03.98E+03.35E+03.35E+03   C>.15A    9.8    9.8  0.85  0.85

X66 791L-J918 EXHA   0.43   0.0   -71.66   8.34  50.28  .25E+03.96E+03.34E+03.34E+03   C>.15A    9.8    9.8  0.85  0.85

X67 691L-J925 EXH5   0.37   0.0   -61.15  -4.89 -45.17  .25E+03.99E+03.35E+03.35E+03   C>.15A    9.8    9.8  0.85  0.85

节点冲剪应力校核

根据API RP 2A-WSD/AISC（校核规范名称通过结果文件读取可获得）进行冲剪应力校核。对极端风暴条件容许应力增加三分之一。冲剪应力校核结果表明，所有节点冲剪UC均小于1.0，满足规范要求。所有节点冲剪应力比结果如下：

（以下内容来自结果文件读取）

* * J O I N T   C A N   S U M M A R Y * *

(UNITY CHECK ORDER)

**************** ORIGINAL *************     ***************** DESIGN **************

JOINT      DIAMETER   THICKNESS  YLD STRS     UC       DIAMETER   THICKNESS  YLD STRS     UC

(CM)       (CM)      (N/MM2)                (CM)       (CM)      (N/MM2)

219L      198.100      6.000    355.005    0.833      198.100      6.000    355.005    0.833

299L      198.100      6.000    355.005    0.824      198.100      6.000    355.005    0.824

J911       91.400      3.000    355.005    0.819       91.400      3.000    355.005    0.819

J910       91.400      3.000    355.005    0.783       91.400      3.000    355.005    0.783

J908       91.400      3.000    355.005    0.752       91.400      3.000    355.005    0.752

191L      198.100      5.000    355.005    0.725      198.100      5.000    355.005    0.725

J909       91.400      3.000    355.005    0.723       91.400      3.000    355.005    0.723

199L      198.100      5.000    355.005    0.703      198.100      5.000    355.005    0.703

J915       76.200      3.000    355.005    0.697       76.200      3.000    355.005    0.697

J914       76.200      3.000    355.005    0.695       76.200      3.000    355.005    0.695

J502       91.400      3.800    355.005    0.694       91.400      3.800    355.005    0.694

111L      198.100      5.000    355.005    0.692      198.100      5.000    355.005    0.692

119L      198.100      5.000    355.005    0.680      198.100      5.000    355.005    0.680

J200       91.400      2.500    355.005    0.662       91.400      2.500    355.005    0.662

J912       76.200      3.000    355.005    0.657       76.200      3.000    355.005    0.657

J503       91.400      3.800    355.005    0.656       91.400      3.800    355.005    0.656

J701       60.100      1.750    355.005    0.646       60.100      1.750    355.005    0.646

599L      167.600      5.500    355.005    0.636      167.600      5.500    355.005    0.636

J913       76.200      3.000    355.005    0.632       76.200      3.000    355.005    0.632

J907       91.400      3.800    355.005    0.614       91.400      3.800    355.005    0.614

J903       91.400      3.800    355.005    0.607       91.400      3.800    355.005    0.607

519L      167.600      5.500    355.005    0.607      167.600      5.500    355.005    0.607

291L      198.100      6.000    355.005    0.585      198.100      6.000    355.005    0.585

211L      198.100      6.000    355.005    0.579      198.100      6.000    355.005    0.579

399L      198.100      5.000    355.005    0.574      198.100      5.000    355.005    0.574

J105       76.200      2.500    355.005    0.573       76.200      2.500    355.005    0.573

319L      198.100      5.000    355.005    0.554      198.100      5.000    355.005    0.554

391L      198.100      5.000    355.005    0.551      198.100      5.000    355.005    0.551

J906       91.400      3.800    355.005    0.550       91.400      3.800    355.005    0.550

J107       76.200      2.500    355.005    0.549       76.200      2.500    355.005    0.549

311L      198.100      5.000    355.005    0.548      198.100      5.000    355.005    0.548

J904       91.400      3.800    355.005    0.542       91.400      3.800    355.005    0.542

J902       91.400      3.800    355.005    0.539       91.400      3.800    355.005    0.539

J106       76.200      2.500    355.005    0.521       76.200      2.500    355.005    0.521

419L      198.100      6.000    355.005    0.519      198.100      6.000    355.005    0.519

J905       91.400      3.800    355.005    0.517       91.400      3.800    355.005    0.517

499L      198.100      6.000    355.005    0.515      198.100      6.000    355.005    0.515

J104       76.200      2.500    355.005    0.513       76.200      2.500    355.005    0.513

J404       91.400      3.800    355.005    0.467       91.400      3.800    355.005    0.467

J207       91.400      3.200    355.005    0.464       91.400      3.200    355.005    0.464

J602       61.000      2.500    355.005    0.463       61.000      2.500    355.005    0.463

J918       75.000      3.200    355.005    0.459       75.000      3.200    355.005    0.459

J900      106.700      4.500    355.005    0.458      106.700      4.500    355.005    0.458

J505       91.400      3.800    355.005    0.452       91.400      3.800    355.005    0.452

J901      106.700      4.500    355.005    0.451      106.700      4.500    355.005    0.451

591L      167.600      5.500    355.005    0.447      167.600      5.500    355.005    0.447

691L      167.600      5.500    355.005    0.447      167.600      5.500    355.005    0.447

J405       91.400      3.800    355.005    0.443       91.400      3.800    355.005    0.443

611L      167.600      5.500    355.005    0.428      167.600      5.500    355.005    0.428

619L      167.600      5.500    355.005    0.424      167.600      5.500    355.005    0.424

699L      167.600      5.500    355.005    0.424      167.600      5.500    355.005    0.424

511L      167.600      5.500    355.005    0.421      167.600      5.500    355.005    0.421

719L      136.000      5.400    355.005    0.418      136.000      5.400    355.005    0.418

J923       91.400      1.900    355.005    0.411       91.400      1.900    355.005    0.411

491L      198.100      6.000    355.005    0.402      198.100      6.000    355.005    0.402

J922       91.400      1.900    355.005    0.395       91.400      1.900    355.005    0.395

799L      136.000      5.400    355.005    0.387      136.000      5.400    355.005    0.387

411L      198.100      6.000    355.005    0.385      198.100      6.000    355.005    0.385

J919       75.000      3.200    355.005    0.376       75.000      3.200    355.005    0.376

J916       75.000      3.200    355.005    0.375       75.000      3.200    355.005    0.375

791L      136.000      5.400    355.005    0.369      136.000      5.400    355.005    0.369

J920       91.400      1.900    355.005    0.369       91.400      1.900    355.005    0.369

J917       75.000      3.200    355.005    0.356       75.000      3.200    355.005    0.356

711L      136.000      5.400    355.005    0.352      136.000      5.400    355.005    0.352

J921       91.400      1.900    355.005    0.351       91.400      1.900    355.005    0.351

J706       60.100      2.050    355.005    0.340       60.100      2.050    355.005    0.340

J102       76.200      2.500    355.005    0.324       76.200      2.500    355.005    0.324

J103       76.200      2.500    355.005    0.323       76.200      2.500    355.005    0.323

J601       61.000      2.500    355.005    0.318       61.000      2.500    355.005    0.318

J101       76.200      2.500    355.005    0.318       76.200      2.500    355.005    0.318

J100       76.200      2.500    355.005    0.317       76.200      2.500    355.005    0.317

J600       61.000      2.500    355.005    0.307       61.000      2.500    355.005    0.307

J201       91.400      3.800    355.005    0.306       91.400      3.800    355.005    0.306

J202       91.400      3.800    355.005    0.295       91.400      3.800    355.005    0.295

J603       61.000      2.500    355.005    0.273       61.000      2.500    355.005    0.273

K11L      198.100      6.500    355.005    0.272      198.100      6.500    355.005    0.272

L91L      198.100      6.500    355.005    0.269      198.100      6.500    355.005    0.269

J303       76.200      2.500    355.005    0.256       76.200      2.500    355.005    0.256

J401       91.400      3.200    355.005    0.253       91.400      3.200    355.005    0.253

J604       61.000      2.500    355.005    0.236       61.000      2.500    355.005    0.236

J302       76.200      2.500    355.005    0.231       76.200      2.500    355.005    0.231

J707       60.100      2.050    355.005    0.222       60.100      2.050    355.005    0.222

J403      106.700      5.000    355.005    0.220      106.700      5.000    355.005    0.220

J402      106.700      5.000    355.005    0.220      106.700      5.000    355.005    0.220

J511       91.400      2.500    355.005    0.216       91.400      2.500    355.005    0.216

J510       91.400      2.500    355.005    0.215       91.400      2.500    355.005    0.215

J400       76.200      3.000    355.005    0.204       76.200      3.000    355.005    0.204

J305       61.000      2.500    355.005    0.196       61.000      2.500    355.005    0.196

J304       61.000      2.500    355.005    0.184       61.000      2.500    355.005    0.184

J504       91.400      3.800    355.005    0.180       91.400      3.800    355.005    0.180

J605       50.800      2.200    355.005    0.175       50.800      2.200    355.005    0.175

J709       49.900      1.750    355.005    0.163       49.900      1.750    355.005    0.163

J700       60.100      2.050    355.005    0.155       60.100      2.050    355.005    0.155

J708       49.900      1.750    355.005    0.152       49.900      1.750    355.005    0.152

J704       60.100      1.750    355.005    0.142       60.100      1.750    355.005    0.142

J500       61.000      2.500    355.005    0.105       61.000      2.500    355.005    0.105

J301       76.200      2.500    355.005    0.099       76.200      2.500    355.005    0.099

J300       76.200      2.500    355.005    0.094       76.200      2.500    355.005    0.094

J501       61.000      2.500    355.005    0.093       61.000      2.500    355.005    0.093

J506       61.000      2.500    355.005    0.092       61.000      2.500    355.005    0.092

J507       61.000      2.500    355.005    0.089       61.000      2.500    355.005    0.089

J606       50.800      2.200    355.005    0.079       50.800      2.200    355.005    0.079

J204      106.700      5.000    355.005    0.073      106.700      5.000    355.005    0.073

J203      106.700      5.000    355.005    0.068      106.700      5.000    355.005    0.068

J509       61.000      2.500    355.005    0.065       61.000      2.500    355.005    0.065

J508       61.000      2.500    355.005    0.065       61.000      2.500    355.005    0.065

J206       91.400      3.800    355.005    0.052       91.400      3.800    355.005    0.052

J205       91.400      3.800    355.005    0.047       91.400      3.800    355.005    0.047

桩基承载力及桩应力校核

桩的极限承载能力参考原详细设计数据, 承载力安全系数如下。

（以下内容来自结果文件读取）

极端风暴工况：

| 桩头ID | 桩基承载能力(kN) |  | 桩自重 （kN） | 设计载荷 |  |  |  | 安全系数 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 抗压 | 抗拔 |  | 工况 | 压力(kN) | 工况 | 拉力(kN) | 抗压 | 抗拔 |
| P101 | 110887 | 117019 | 6864.3 | EL37 | 58717.7 | LL13 | 12220.3 | 1.69 | 21.85 |
| P201 | 110887 | 117019 | 6864.3 | EL36 | 62748.6 | LL12 | 25286.5 | 1.59 | 6.35 |
| P301 | 110887 | 117019 | 6864.3 | EL36 | 47580.7 | LL12 | 3380.7 | 2.04 | - |
| P104 | 110887 | 117019 | 6864.3 | EL47 | 64616.7 | LL13 | 16365.0 | 1.55 | 12.32 |
| P204 | 110887 | 117019 | 6864.3 | EL48 | 64844.7 | LL14 | 28511.9 | 1.55 | 5.41 |
| P304 | 110887 | 117019 | 6864.3 | EL48 | 48443.1 | LL15 | 2051.5 | 2.00 | - |
| P105 | 110887 | 117019 | 6864.3 | EL13 | 58159.9 | LL17 | 12169.6 | 1.71 | 22.06 |
| P205 | 110887 | 117019 | 6864.3 | EL14 | 62558.9 | LL18 | 25535.9 | 1.60 | 6.27 |
| P305 | 110887 | 117019 | 6864.3 | EL14 | 47178.9 | LL18 | 3051.8 | 2.05 | - |
| P108 | 110887 | 117019 | 6864.3 | EL23 | 64273.2 | LL17 | 15782.7 | 1.56 | 13.12 |
| P208 | 110887 | 117019 | 6864.3 | EL22 | 64449.3 | LL16 | 27292.4 | 1.55 | 5.73 |
| P308 | 110887 | 117019 | 6864.3 | EL22 | 47427.0 | LL15 | 1313.4 | 2.04 | - |

操作工况：

| 桩头ID | 桩基承载能力(kN) |  | 桩自重 （kN） | 设计载荷 |  |  |  | 安全系数 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 抗压 | 抗拔 |  | 工况 | 压力(kN) | 工况 | 拉力(kN) | 抗压 | 抗拔 |
| P101 | 110887 | 117019 | 6864.3 | OL37 | 37389.0 | - | - | 2.51 | - |
| P201 | 110887 | 117019 | 6864.3 | OL36 | 35407.2 | - | - | 2.62 | - |
| P301 | 110887 | 117019 | 6864.3 | OL36 | 31457.0 | - | - | 2.89 | - |
| P104 | 110887 | 117019 | 6864.3 | OL47 | 40428.6 | - | - | 2.34 | - |
| P204 | 110887 | 117019 | 6864.3 | OL48 | 36717.9 | - | - | 2.54 | - |
| P304 | 110887 | 117019 | 6864.3 | OL48 | 32869.2 | - | - | 2.79 | - |
| P105 | 110887 | 117019 | 6864.3 | OL13 | 36638.8 | - | - | 2.55 | - |
| P205 | 110887 | 117019 | 6864.3 | OL14 | 34679.9 | - | - | 2.67 | - |
| P305 | 110887 | 117019 | 6864.3 | OL14 | 30875.3 | - | - | 2.94 | - |
| P108 | 110887 | 117019 | 6864.3 | OL23 | 39901.0 | - | - | 2.37 | - |
| P208 | 110887 | 117019 | 6864.3 | OL22 | 36242.6 | - | - | 2.57 | - |
| P308 | 110887 | 117019 | 6864.3 | OL22 | 32245.5 | - | - | 2.84 | - |

从以上结果可知：极端工况下平台桩承载力安全系数最小为1.51，大于1.5；操作工况下平台桩承载力安全系数最小为2.87，大于2.0，桩基承载力安全系数满足规范要求。（根据结果更新最小安全系数）

桩应力UC最大为0.719，小于1.0，满足规范。桩应力计算结果如下.（更新最大UC值，以下内容来自结果文件读取）

* *  P I L E  G R O U P  S U M M A R Y  * *

GROUP ID = PL1

DISTANCE  ***** DEFLECTIONS *****  ***** INTERNAL LOADS *****  ********** STRESSES **********    PILE   CRITICAL  MAXIMUM

FROM                              BENDING             AXIAL  BENDING  AXIAL   SHEAR   COMB.    HEAD     LOAD     UNITY

PILEHEAD LATERAL   AXIAL   ROT.      MOMENT    SHEAR     LOAD  STRESS  STRESS  STRESS  STRESS     ID      CASE     CHECK

M        CM      CM     RAD        KN-M      KN        KN   N/MM2   N/MM2   N/MM2   N/MM2

0.0     9.018   2.597 0.00357    29623.3   4654.7 -24494.4  162.75  -68.19   25.91 -230.93   131S     EXL3      0.719

0.7     8.743   2.572 0.00407    26254.6   4660.4 -24498.5  144.24  -68.20   25.95 -212.44   131S     EXL3      0.665

1.4     8.432   2.548 0.00451    22879.1   4668.8 -24501.0  125.69  -68.20   25.99 -193.90   131S     EXL3      0.610

2.2     8.092   2.523 0.00489    19502.6   4652.5 -24491.6  107.14  -68.18   25.90 -175.32   131S     EXL3      0.556

2.9     7.725   2.498 0.00521    16157.5   4612.0 -24474.5   88.77  -68.13   25.68 -156.90   131S     EXL3      0.502

3.6     7.050   2.557 0.00528    12512.4   4459.4 -25023.8   68.74  -69.66   24.83 -138.40   125S     EXL4      0.448

4.3     6.659   2.532 0.00547     9336.3   4387.7 -24987.2   51.29  -69.56   24.43 -120.85   125S     EXL4      0.396

5.1     6.257   2.506 0.00561     6256.7   4283.8 -24935.9   34.37  -69.42   23.85 -103.79   125S     EXL4      0.346

5.8     5.987   3.053 0.00642     3300.1   3730.3 -28598.4   18.13  -79.61   20.77  -97.74   129S     EXL2      0.334

6.5     5.523   3.024 0.00635     5429.0   3582.5 -28518.4   29.83  -79.39   19.95 -109.21   129S     EXL2      0.368

7.2     5.066   2.996 0.00623     7775.7   3309.1 -28431.9   42.72  -79.15   18.42 -121.87   129S     EXL2      0.405

8.0     4.620   2.967 0.00607     9933.6   2714.1 -28336.5   54.57  -78.88   15.11 -133.46   129S     EXL2      0.439

8.7     4.187   2.939 0.00587    11554.7   2051.1 -28230.3   63.48  -78.59   11.42 -142.07   129S     EXL2      0.464

9.4     3.769   2.910 0.00564    12805.2   1689.7 -28119.4   70.35  -78.28    9.41 -148.63   129S     EXL2      0.483

10.2     3.368   2.882 0.00539    13926.7   1498.2 -28007.0   76.51  -77.96    8.34 -154.48   129S     EXL2      0.500

10.9     2.987   2.854 0.00513    14915.1   1307.2 -27890.9   81.94  -77.64    7.28 -159.58   129S     EXL2      0.515

11.6     2.625   2.826 0.00484    15770.4   1118.8 -27771.1   86.64  -77.31    6.23 -163.95   129S     EXL2      0.528

12.3     2.285   2.798 0.00454    16493.4    933.5 -27647.6   90.61  -76.96    5.20 -167.58   129S     EXL2      0.538

13.1     1.967   2.770 0.00423    17085.6    751.5 -27520.6   93.86  -76.61    4.18 -170.48   129S     EXL2      0.546

13.8     1.672   2.742 0.00390    17548.5    573.3 -27390.0   96.41  -76.25    3.19 -172.66   129S     EXL2      0.553

14.5     1.401   2.714 0.00357    17884.0    290.7 -27252.2   98.25  -75.86    1.62 -174.12   129S     EXL2      0.557

15.2     1.154   2.687 0.00324    17922.4    193.1 -27103.8   98.46  -75.45    1.08 -173.91   129S     EXL2      0.556

15.9     0.932   2.660 0.00291    17658.2    551.5 -26945.0   97.01  -75.01    3.07 -172.02   129S     EXL2      0.550

16.7     0.733   2.633 0.00258    17135.6    872.6 -26775.7   94.14  -74.54    4.86 -168.68   129S     EXL2      0.540

17.4     0.557   2.606 0.00227    16399.3   1136.0 -26596.1   90.09  -74.04    6.32 -164.13   129S     EXL2      0.526

18.1     0.404   2.576 0.00197    15492.4   1342.7 -26405.1   92.96  -80.66    8.20 -173.62   129S     EXL2      0.563

18.9     0.273   2.547 0.00167    14455.2   1510.4 -26210.5   86.74  -80.07    9.23 -166.81   129S     EXL2      0.542

19.6     0.163   2.518 0.00138    13304.3   1660.8 -26016.8   79.83  -79.48   10.15 -159.31   129S     EXL2      0.519

20.3     0.073   2.490 0.00113    12048.4   1766.5 -25820.5   72.30  -78.88   10.79 -151.17   129S     EXL2      0.495

21.0     0.012   2.461 0.00089    10743.8   1802.2 -25621.6   64.47  -78.27   11.01 -142.74   129S     EXL2      0.469

21.8     0.059   2.433 0.00069     9436.0   1782.0 -25420.1   56.62  -77.65   10.89 -134.27   129S     EXL2      0.443

22.5     0.102   2.405 0.00051     8160.5   1718.7 -25216.1   48.97  -77.03   10.50 -126.00   129S     EXL2      0.418

23.2     0.132   2.377 0.00035     6944.4   1623.3 -25009.6   41.67  -76.40    9.92 -118.07   129S     EXL2      0.394

23.9     0.153   2.349 0.00023     5807.1   1505.5 -24800.6   34.85  -75.76    9.20 -110.61   129S     EXL2      0.372

24.6     0.165   2.322 0.00012     4761.9   1373.2 -24589.1   28.57  -75.11    8.39 -103.69   129S     EXL2      0.351

25.4     0.169   2.295 0.00003     3816.5   1233.9 -24375.4   22.90  -74.46    7.54  -97.36   129S     EXL2      0.331

26.1     0.169   2.268 0.00004     2973.2   1092.8 -24160.5   17.84  -73.81    6.68  -91.65   129S     EXL2      0.314

26.8     0.164   2.241 0.00009     2232.5    953.0 -23945.7   13.40  -73.15    5.82  -86.55   129S     EXL2      0.298

27.6     0.155   2.215 0.00013     1592.0    818.1 -23730.7    9.55  -72.49    5.00  -82.05   129S     EXL2      0.284

28.3     0.145   2.189 0.00016     1047.1    691.1 -23515.7    6.28  -71.84    4.22  -78.12   129S     EXL2      0.272

29.0     0.132   2.163 0.00018      591.7    573.6 -23300.7    3.55  -71.18    3.50  -74.73   129S     EXL2      0.262

29.7     0.119   2.137 0.00018      220.8    466.8 -23085.6    1.32  -70.52    2.85  -71.85   129S     EXL2      0.253

30.4     0.106   2.112 0.00018      102.3    371.1 -22870.5    0.61  -69.86    2.27  -70.48   129S     EXL2      0.248

31.2     0.092   2.087 0.00018      328.0    286.7 -22655.5    1.97  -69.21    1.75  -71.18   129S     EXL2      0.250

31.9     0.079   2.062 0.00017      505.6    213.4 -22441.0    3.03  -68.55    1.30  -71.59   129S     EXL2      0.251

32.6     0.067   2.037 0.00016      634.9    150.6 -22227.1    3.81  -67.90    0.92  -71.71   129S     EXL2      0.251

33.4     0.056   2.012 0.00015      722.8     97.8 -22013.8    4.34  -67.25    0.60  -71.58   129S     EXL2      0.250

34.1     0.046   1.988 0.00013      775.8     54.0 -21801.2    4.66  -66.60    0.33  -71.25   129S     EXL2      0.249

34.8     0.037   1.964 0.00012      800.2     18.8 -21589.2    4.80  -65.95    0.11  -70.75   129S     EXL2      0.247

35.5     0.029   1.940 0.00010      801.4     12.0 -21377.9    4.81  -65.31    0.07  -70.11   129S     EXL2      0.245

36.2     0.023   1.924 0.00008      784.3     33.7 -21171.3    3.25  -43.64    0.14  -46.89   129S     EXL2      0.163

37.0     0.017   1.909 0.00007      753.0     50.8 -20969.1    3.12  -43.22    0.21  -46.34   129S     EXL2      0.161

37.7     0.012   1.893 0.00006      710.9     63.4 -20766.9    2.95  -42.81    0.26  -45.75   129S     EXL2      0.159

38.4     0.008   1.878 0.00005      661.1     72.3 -20564.7    2.74  -42.39    0.30  -45.13   129S     EXL2      0.157

39.1     0.005   1.862 0.00004      606.2     77.8 -20362.6    2.51  -41.97    0.32  -44.49   129S     EXL2      0.155

39.9     0.002   1.847 0.00004      548.3     80.7 -20160.4    2.27  -41.56    0.33  -43.83   129S     EXL2      0.153

40.6     0.001   1.832 0.00003      489.2     81.4 -19958.3    2.03  -41.14    0.34  -43.17   129S     EXL2      0.151

41.3     0.002   1.817 0.00002      430.3     80.3 -19756.2    1.78  -40.72    0.33  -42.51   129S     EXL2      0.149

42.0     0.004   1.803 0.00002      372.8     77.6 -19521.4    1.55  -40.24    0.32  -41.78   129S     EXL2      0.146

42.8     0.005   1.788 0.00001      317.8     73.3 -19254.8    1.32  -39.69    0.30  -41.01   129S     EXL2      0.144

43.5     0.005   1.774 0.00001      266.5     68.0 -18989.8    1.10  -39.14    0.28  -40.25   129S     EXL2      0.141

44.2     0.006   1.760 0.00000      219.3     62.0 -18726.5    0.91  -38.60    0.26  -39.51   129S     EXL2      0.139

45.0     0.006   1.746 0.00000      176.6     55.8 -18464.7    0.73  -38.06    0.23  -38.79   129S     EXL2      0.136

45.7     0.006   1.732 0.00000      138.5     49.4 -18204.5    0.57  -37.52    0.20  -38.10   129S     EXL2      0.134

46.4     0.006   1.719 0.00000      104.9     43.2 -17945.8    0.43  -36.99    0.18  -37.43   129S     EXL2      0.132

47.1     0.005   1.705 0.00000       75.9     37.2 -17688.6    0.31  -36.46    0.15  -36.78   129S     EXL2      0.130

47.9     0.005   1.692 0.00001       51.1     31.5 -17433.0    0.21  -35.93    0.13  -36.15   129S     EXL2      0.127

48.6     0.005   1.679 0.00001       30.2     26.3 -17178.8    0.13  -35.41    0.11  -35.54   129S     EXL2      0.125

49.3     0.004   1.667 0.00001       13.2     21.4 -16926.1    0.05  -34.89    0.09  -34.94   129S     EXL2      0.123

50.0     0.004   1.654 0.00001        2.7     17.1 -16674.8    0.01  -34.37    0.07  -34.38   129S     EXL2      0.121

50.8     0.003   1.642 0.00001       12.1     13.3 -16425.0    0.05  -33.86    0.05  -33.91   129S     EXL2      0.120

51.5     0.003   1.630 0.00001       20.3      9.9 -16176.5    0.08  -33.34    0.04  -33.43   129S     EXL2      0.118

52.2     0.002   1.618 0.00001       26.3      7.0 -15929.4    0.11  -32.83    0.03  -32.94   129S     EXL2      0.116

52.9     0.002   1.606 0.00001       30.4      4.6 -15683.6    0.13  -32.33    0.02  -32.45   129S     EXL2      0.114

53.6     0.002   1.594 0.00000       32.9      2.5 -15439.2    0.14  -31.82    0.01  -31.96   129S     EXL2      0.113

54.4     0.001   1.583 0.00000       34.0      0.8 -15196.1    0.14  -31.32    0.00  -31.46   129S     EXL2      0.111

55.1     0.001   1.571 0.00000       34.0      0.6 -14954.3    0.14  -30.82    0.00  -30.97   129S     EXL2      0.109

55.8     0.001   1.560 0.00000       33.2      1.6 -14713.7    0.14  -30.33    0.01  -30.47   129S     EXL2      0.107

56.5     0.001   1.549 0.00000       31.7      2.4 -14474.4    0.13  -29.84    0.01  -29.97   129S     EXL2      0.106

57.3     0.000   1.539 0.00000       29.7      3.0 -14236.4    0.12  -29.34    0.01  -29.47   129S     EXL2      0.104

58.0     0.000   1.528 0.00000       27.3      3.4 -13999.5    0.11  -28.86    0.01  -28.97   129S     EXL2      0.102

58.7     0.000   1.518 0.00000       24.8      3.6 -13763.8    0.10  -28.37    0.01  -28.47   129S     EXL2      0.100

59.5     0.000   1.508 0.00000       22.1      3.7 -13529.3    0.09  -27.89    0.02  -27.98   129S     EXL2      0.099

60.2     0.000   1.498 0.00000       19.4      3.7 -13281.5    0.08  -27.38    0.02  -27.46   129S     EXL2      0.097

60.9     0.000   1.488 0.00000       16.7      3.6 -13020.5    0.07  -26.84    0.01  -26.91   129S     EXL2      0.095

61.6     0.000   1.478 0.00000       14.2      3.4 -12760.7    0.06  -26.30    0.01  -26.36   129S     EXL2      0.093

62.4     0.000   1.469 0.00000       11.8      3.1 -12502.0    0.05  -25.77    0.01  -25.82   129S     EXL2      0.091

63.1     0.000   1.459 0.00000        9.6      2.9 -12244.6    0.04  -25.24    0.01  -25.28   129S     EXL2      0.089

63.8     0.000   1.450 0.00000        7.6      2.5 -11988.3    0.03  -24.71    0.01  -24.74   129S     EXL2      0.087

64.5     0.000   1.442 0.00000        5.9      2.2 -11733.2    0.02  -24.19    0.01  -24.21   129S     EXL2      0.085

65.2     0.000   1.433 0.00000        4.4      1.9 -11479.1    0.02  -23.66    0.01  -23.68   129S     EXL2      0.084

66.0     0.000   1.424 0.00000        3.1      1.6 -11226.2    0.01  -23.14    0.01  -23.15   129S     EXL2      0.082

66.7     0.000   1.416 0.00000        2.1      1.3 -10974.2    0.01  -22.62    0.01  -22.63   129S     EXL2      0.080

67.4     0.000   1.408 0.00000        1.2      1.1 -10723.4    0.01  -22.10    0.00  -22.11   129S     EXL2      0.078

68.2     0.000   1.400 0.00000        0.5      0.8 -10473.5    0.00  -21.59    0.00  -21.59   129S     EXL2      0.076

68.9     0.000   1.392 0.00000        0.1      0.6 -10224.6    0.00  -21.08    0.00  -21.08   129S     EXL2      0.074

69.6     0.000   1.378 0.00000        0.4      0.5  -9970.6    0.00  -39.85    0.00  -39.85   129S     EXL2      0.141

70.3     0.000   1.363 0.00000        0.7      0.3  -9711.9    0.01  -38.81    0.00  -38.82   129S     EXL2      0.137

71.0     0.000   1.350 0.00000        0.9      0.2  -9454.9    0.01  -37.79    0.00  -37.79   129S     EXL2      0.133

71.8     0.000   1.336 0.00000        1.0      0.1  -9199.6    0.01  -36.77    0.00  -36.77   129S     EXL2      0.130

72.5     0.000   1.323 0.00000        1.1      0.0  -8882.6    0.01  -35.50    0.00  -35.51   129S     EXL2      0.125

73.2     0.000   1.310 0.00000        1.1      0.1  -8502.8    0.01  -33.98    0.00  -33.99   129S     EXL2      0.120

74.0     0.000   1.298 0.00000        1.0      0.1  -8123.1    0.01  -32.46    0.00  -32.47   129S     EXL2      0.115

74.7     0.000   1.287 0.00000        0.9      0.2  -7743.4    0.01  -30.95    0.00  -30.95   129S     EXL2      0.109

75.4     0.000   1.276 0.00000        0.8      0.2  -7363.6    0.01  -29.43    0.00  -29.43   129S     EXL2      0.104

76.1     0.000   1.265 0.00000        0.6      0.2  -6983.9    0.00  -27.91    0.00  -27.92   129S     EXL2      0.099

76.8     0.000   1.256 0.00000        0.5      0.2  -6604.1    0.00  -26.39    0.00  -26.40   129S     EXL2      0.093

77.6     0.000   1.246 0.00000        0.4      0.2  -6224.4    0.00  -24.88    0.00  -24.88   129S     EXL2      0.088

78.3     0.000   1.238 0.00000        0.3      0.1  -5808.2    0.00  -23.21    0.00  -23.21   129S     EXL2      0.082

79.0     0.000   1.229 0.00000        0.2      0.1  -5355.6    0.00  -21.40    0.00  -21.40   129S     EXL2      0.076

79.8     0.000   1.222 0.00000        0.1      0.1  -4902.9    0.00  -19.59    0.00  -19.60   129S     EXL2      0.069

80.5     0.000   1.215 0.00000        0.0      0.1  -4450.3    0.00  -17.79    0.00  -17.79   129S     EXL2      0.063

81.2     0.000   1.209 0.00000        0.0      0.0  -3997.6    0.00  -15.98    0.00  -15.98   129S     EXL2      0.056

81.9     0.000   1.204 0.00000        0.0      0.0  -3545.0    0.00  -14.17    0.00  -14.17   129S     EXL2      0.050

82.7     0.000   1.199 0.00000        0.0      0.0  -3173.8    0.00  -12.68    0.00  -12.68   129S     EXL2      0.045

83.4     0.000   1.194 0.00000        0.0      0.0  -2882.2    0.00  -11.52    0.00  -11.52   129S     EXL2      0.041

84.1     0.000   1.192 0.00000        0.0      0.0  -2590.2    0.00   -7.21    0.00   -7.21   129S     EXL2      0.025

84.8     0.000   1.189 0.00000        0.0      0.0  -2297.6    0.00   -6.40    0.00   -6.40   129S     EXL2      0.023

85.5     0.000   1.187 0.00000        0.0      0.0  -2001.3    0.00   -5.57    0.00   -5.57   129S     EXL2      0.020

86.3     0.000   1.185 0.00000        0.0      0.0  -1701.3    0.00   -4.74    0.00   -4.74   129S     EXL2      0.017

87.0     0.000   1.184 0.00000        0.0      0.0   -775.2    0.00   -2.16    0.00   -2.16   129S     EXL2      0.008